- S-57 APPENDIX B.1

- Annex A - Use of the Object Catalogue for ENC

This document must only be used with Edition 2.0 of S-57 Appendix B.1 ENC PRODUCT SPECIFICATION

For Corrections since November 1999, see ADDENDUM at the end of the Document

Edition 2.0

Page intentionally left blank

- CONTENTS

1.	  Introduction	A.1

1.1	 General	A.1

1.2	 Presentation of the document	A.1

1.3	 Use of language	A.1

2.	  General rules	A.1

2.1	 Cartographic framework	A.1

2.1.1	 Horizontal datum	A.1

2.1.2	 Vertical datum	A.2

2.1.3	 Sounding datum	A.2

2.1.4	 Units	A.3

2.1.5	 Dates	A.3

2.1.5.1	  Seasonal Objects	A.3

2.1.6	 Times	A.3

2.2	 Data quality description	A.3

2.2.1	 Production information and up-to-dateness	A.3

2.2.2	 Up-to-dateness information	A.3

2.2.3	 Quality and accuracy of bathymetric data	A.3

2.2.3.1	  Quality of bathymetric data	A.4

2.2.3.2	  Survey reliability	A.5

2.2.3.4	  Sounding accuracy	A.5

2.2.3.5	  Technique of sounding measurement	A.5

2.2.4	 Accuracy	A.5

2.2.4.1	  Quality of positions	A.5

2.2.4.2	  Horizontal accuracy	A.5

2.2.4.3	  Vertical accuracy	A.6

2.2.5	 Source of data	A.6

2.2.5.1	   Source of bathymetric data	A.6

2.2.5.2	   Source of other data	A.6

2.2.6	 Compilation scale	A.6

2.2.7	 Use of the attribute SCAMIN	A.6

2.3	 Textual information	A.7

2.4	 Colours and colour patterns	A.7

2.5	 Reference to other publications	A.8

2.6 	 Updating	A.8

2.6.1	 Issuing updates in advance	A.8

2.7	 Multiple objects	A.8

3.	  Time Varying Objects	A.9

3.1	 Magnetic data (see M4 - §270)	A.9

3.1.1	 Magnetic variation	A.9

3.1.2	 Abnormal magnetic variation (see M4 - §274)	A.10

3.2	 Tidal data (see M4 - §406)	A.10

3.2.1	 Time series data	A.10

3.2.2	 Prediction by harmonic methods	A.10

3.2.3	 Prediction by non-harmonic methods	A.11

3.3.1	 Tidal stream (flood/ebb)	A.11

3.3.2	 Tidal stream time series	A.11

3.3.3	 Prediction by harmonic methods	A.11

3.3.4	 Prediction by non-harmonic methods	A.12

3.3.5	 Tidal stream panels	A.12

3.4	 Current data (see M4 - §408)	A.12

4.	  Topography	A.12

4.1	 Land area	A.12

4.2	 Vertical measurements (see M4 - §302 and §303)	A.13

4.2.1	 Vertical datum	A.13

4.2.2	 Heights and elevations	A.13

4.3	 Control points (see M4 - §304 to §306)	A.13

4.4	 Distance marks (see M4 - §307)	A.13

4.5	 Coastline	A.14

4.5.1	 Natural coastline (see M4 - §312)	A.14

4.5.2	 Artificial coastline (see M4 - §313 and §320)	A.14

4.6	 Harbour installations	A.15

4.6.1	 Harbour facilities	A.15

4.6.2	 Berths (see M4 - §323)	A.15

4.6.3	 Harbour offices (see M4 - §325)	A.15

4.6.4	 Checkpoints	A.16

4.6.5 	 Small craft facilities	A.16

4.6.6	 Docks	A.16

4.6.6.1	  Dry docks (see M4 - §326.1)	A.16

4.6.6.2	  Floating docks (see M4 - §326.2)	A.16

4.6.6.3	  Tidal and non-tidal basins (see M4 - §326.3 and §326.4)	A.17

4.6.6.4	  Gates (see M4 - §326.5 to §326.7)	A.17

4.6.6.5	  Locks	A.17

4.6.6.6	  Gridiron (see M4 - §326.8)	A.18

4.6.7	 Mooring / warping facilities and pontoons	A.18

4.6.7.1	  Mooring / warping facilities (see M4 - §327)	A.18

4.6.7.2	  Piles	A.18

4.6.7.3	  Pontoons (see M4 - §326.9)	A.18

4.6.8	 Hulks	A.19

4.6.9	 Dockside buildings and structures	A.19

4.6.9.1	  Transit sheds and warehouses (see M4 - §328.1)	A.19

4.6.9.2	  Timber yards (see M4 - §328.2)	A.19

4.6.9.3	  Cranes (see M4 - §328.3)	A.19

4.6.10	  Works in progress and projected (see M4 - §329)	A.19

4.7	 Natural features (see M4 - §350)	A.20

4.7.1	  Natural sceneries	.A.20

4.7.4	 Dunes, sand hills (see M4 - §312.3)	A.20

4.7.5	 Cliffs (see M4 - §312.1)	A.21

4.7.6	 Rivers (see M4 - §353)	A.21

4.7.7	 Rapids, waterfalls (see M4 - §353.5)	A.21

4.7.7.1	  Rapids	A.21

4.7.7.2	  Waterfalls	A.22

4.7.8	 Lakes (see M4 - §353.6)	A.22

4.7.9	 Saltpans (see M4 - §353.7)	A.22

4.7.10	  Glaciers (see M4 - §353.8)	A.22

4.7.11	  Vegetation (see M4 - §352.4 and §354)	A.22

4.7.12	  Lava flow (see M4 - §355)	A.22

4.8	 Artificial features	A.23

4.8.1	 Canals (see M4 - §361)	A.23

4.8.2	 Railways (see M4 - §362)	A.23

4.8.3	 Tunnels (see M4 - §363.1)	A.23

4.8.4	 Cuttings and embankments (see M4 - §363.2 and §364.1)	A.23

4.8.5	 Dams (see M4 - §364.2)	A.24

4.8.6	 Flood barrages (see M4 - §326.7)	A.24

4.8.7	 Dykes (see M4 - §313.1)	A.24

4.8.8	 Roads and tracks (see M4 - §365)	A.24

4.8.9	 Causeways	A.25

4.8.12	  Airfields (see M4 - §366)	A.25

4.8.13	 Production and storage areas (see M4 - §367)	A.26

4.8.14	 Built-up areas (see M4 - §370)	A.26

4.8.15	 Buildings, landmarks, tanks, silos (see M4 - §370 to §378)	A.30

4.8.17	 Fortified structures (see M4 - §379)	A.30

4.8.18	 Pylons and bridge supports (see M4 - §381 and §382)	A.30

4.8.19	 Oil barriers	A.30

4.8.20	 Views and sketches, viewpoints	A.30

4.8.21 Signs and Notice boards	A.32

5.1	 Sounding datum	A.32

5.2	 Depth contours (see M4 - §411)	A.32

5.3	 Soundings (see M4 - §412)	A.32

5.4	 Depth areas	A.33

5.4.1	 Geo object depth area	A.33

5.4.2	 Geometry of depth areas	A.34

5.4.3	 Use of DRVAL1 and DRVAL2 for depth areas in general.	A.34

5.4.4	 Use of DRVAL1 and DRVAL2 for depth areas adjoining non-navigable waterways.	A.37

5.4.5	 Use of DRVAL1 and DRVAL2 for depth areas adjoining constructions

which are always dry	A.38

5.4.6	 Use of DRVAL1 and DRVAL2 for depth areas adjoining a dredged area	A.38

5.4.7	 Use of DRVAL1 and DRVAL2 for dredged areas adjoining dredged areas	A.39

5.4.8	 Rivers, canals, lakes, basins	A.40

5.5	 Dredged areas (see M4 - §414)	A.40

5.6	 Swept areas (see M4 - §415)	A.41

5.7	 Areas of continual change (see M4 - §416)	A.41

5.8	 Areas with inadequate depth information (see M4 - §417)	A.41

5.8.1	 Unsurveyed areas	A.41

5.8.2	 Incompletely surveyed areas	A.42

5.8.3 	 Bathymetry in areas of minimal depiction of detail	A.42

5.8.3.1  	Areas of omitted bathymetry	A.42

5.8.3.2  	Areas of very simplified bathymetry	A.42

6.	  Dangers	A.43

6.1	 Rocks and coral reefs (see M4 - §421)	A.43

6.1.1	 Rocks which do not cover (islets)	A.43

6.1.2	 Rocks which may cover	A.43

6.2	 Wrecks, foul areas and obstructions (see M4 - §422)	A.44

6.2.1	 Wrecks	A.44

6.2.2	 Obstructions and foul areas	A.45

6.3	 Danger lines	A.46

6.3.1	 Danger line around a point danger or an isolated sounding.	A.46

6.3.2	 Danger line limiting an area of wrecks or obstructions	A.46

6.3.3	 Danger line limiting an area through which navigation is not safe (see M4 -§420.1)	A.46

6.4	 Overfalls, races, breakers, eddies (see M4 - §423)	A.46

6.5	 Doubtful dangers (see M4 - §424)	A.47

6.6	 Caution area	A.47

7.	  Nature of the seabed	A.48

7.1 	 Description of the bottom (see M4 - §425)	A.48

7.2	 Special bottom types	A.48

7.2.1	 Sand waves (see M4 - §428.1)	A.48

7.2.2	 Weed - Kelp (see M4 - §428.2)	A.49

7.2.3	 Springs in the seabed (see M4 - §428.3)	A.49

7.2.4	 Tideway	A.49

9.	  Harbour regulations	A.50

9.1	 Regulations within harbour limits	A.50

9.1.1	 Administrative harbour areas (see M4 - §430.1)	A.50

9.1.2	 Speed limits (see M4 - §430.2)	A.50

9.2	 Anchorages and prohibited anchorages; moorings	A.50

9.2.1	 Anchorages (see M4 - §431.1 and 431.3)	A.50

9.2.2	 Anchor berths (see M4 - §431.2)	A.50

9.2.3	 Anchoring restricted (see M4 - §431.4)	A.51

9.2.4	 Mooring buoys (see M4 - §431.5)	A.51

9.2.5	 Mooring trots (see M4 - §431.6)	A.51

9.2.6 	 Anchorage - Relationship	A.51

10.	 Recommended tracks and routes	A.52

10.1	 Leading, clearing and transit lines and recommended tracks (see M4 - §433)	A.52

10.1.1	  Navigation lines and recommended tracks	A.52

10.1.2	  Range systems - Relationship	A.53

10.1.3	  Measured distance	A.54

10.2	  Routeing measures	A.54

10.2.1	  Traffic separation scheme	A.54

10.2.1.1	 Traffic separation scheme lanes (see M4 - §435.1)	A.54

10.2.1.2	 Traffic separation scheme boundaries (see M4 - §435.1)	A.55

10.2.1.3	 Traffic separation lines (see M4 - §435.1)	A.55

10.2.1.4	 Traffic separation zones (see M4 - §435.1)	A.55

10.2.1.5	 Traffic separation scheme crossings (see M4 - §435.1)	A.55

10.2.1.6	 Traffic separation scheme roundabouts (see M4 - §435.1)	A.55

10.2.1.7	 Inshore traffic zones (see M4 - §435.1)	A.56

10.2.1.8	 Precautionary areas (see M4 - §435.2)	A.56

10.2.2	  Deep water routes (see M4 - §435.3)	A.56

10.2.2.1	 Deep water route parts	A.56

10.2.2.2	 Deep water route centrelines	A.56

10.2.3	  Traffic separation scheme systems	A.57

10.2.4	  Recommended routes (see M4 - §435.4)	A.57

10.2.5	  Recommended direction of traffic flow (see M4 - §435.5)	A.57

10.2.6	  Two-way routes (see M4 - §435.6)	A.58

10.2.7	  Areas to be avoided (see M4 - §435.7)	A.58

10.3	  Ferries (see M4 - §438)	A.58

10.4	  Fairways	A.58

11.	  Regulated areas	A.59

11.1	  Restricted areas in general (see M4 - §439.2 to § 439.4)	A.59

11.2	  Maritime jurisdiction areas (see M4 - §440)	A.60

11.2.1	  National territory	A.60

11.2.2	  Custom zones	A.60

11.2.3	  Free port area	A.60

11.2.4	  Territorial sea	A.60

11.2.5	  Contiguous zone	A.61

11.2.6	  Fishery zones	A.61

11.2.7	  Continental shelf	A.61

11.2.8	  Exclusive economic zone	A.61

11.3	  Military practice areas; minefields	A.61

11.3.1	  Military practice areas (see M4 - §441)	A.61

11.3.2	  Submarine transit lanes (see M4 - §441.5)	A.62

11.3.3	  Minefields (see M4 - §441.8)	A.62

11.4	  Dumping grounds (see M4 - §442)	A.62

11.5	  Cables and cable areas	A.62

11.5.1	  Submarine cables (see M4 - §443)	A.62

11.5.2	  Overhead cables (see M4 - §382)	A.62

11.5.3	  Submarine cable areas (see M4 - §443.2)	A.63

11.6	  Pipelines and pipeline areas	A.63

11.6.1	  Pipelines, submarine or on land (see M4 - §377 and §444)	A.63

11.6.2	  Diffusers, cribs	A.63

11.6.3	  Overhead pipelines (see M4 - §383)	A.64

11.6.4	  Pipeline areas (see M4 - §444.2)	A.64

11.7	  Oil and Gas fields	A.64

11.7.1	  Wellheads (see M4 - §445.1, §445.5 and §445.7)	A.64

11.7.2	  Offshore platforms (see M4 - §445.2)	A.64

11.7.3	  Safety zones (see M4 - §445.2)	A.65

11.7.4	  Offshore production area (see M4 - §445.3)	A.65

11.7.5	  Offshore tanker loading systems (see M4 - §445.4)	A.65

11.7.6	  Flare stacks (see M4 - §445.6)	A.65

11.8	  Spoil grounds, dredging areas (see M4 - §446)	A.65

11.9	  Fishing equipment and aquaculture areas	A.65

11.9.1	  Fishing facilities (see M4 - §447.1 to §447.3)	A.65

11.9.2	  Marine farms (see M4 - § 447.4 and §447.6)	A.66

11.9.3	  Fish havens (see M4 - §447.5)	A.66

11.9.4	  Fishing ground	A.66

11.10	 Degaussing ranges (see M4 - §448)	A.66

11.11	 Historic wrecks (see M4 §449.5)	A.66

11.12	 Seaplane landing area (see M4 §449.6)	A.66

11.13	 Various maritime areas	A.67

11.13.1	 Ice areas (see M4 - §449.1)	A.67

11.13.2	 Log ponds (see M4 - §449.2)	A.67

11.13.3	 Incineration area (see M4 - §449.3)	A.67

11.13.4	 Cargo transhipment area (see M4 - §449.4)	A.67

11.14 	 Nature reserves	A.67

12.	  Aids to navigation	A.67

12.1	  Lighthouses, navigational marks - Relationship	A.67

12.1.1	  Geo objects part of navigational aids	A.67

12.1.2	  Relationships	A.68

12.2	  Buoyage system and direction of buoyage (see M4 - §461)	A.68

12.3	  Fixed structures (see M4 - §455)	A.69

12.3.1	  Beacons	A.69

12.3.2	  Lighthouses (see M4 - § 457.3)	A.70

12.3.3	  Daymarks	A.70

12.4	  Floating structures (see M4 - §460)	A.71

12.4.1	  Buoys	A.71

12.4.2	  Light float and light vessel (see M4 - § 462.8 and §474.1)	A.72

12.5	  Fog signals (see M4 - §451)	A.72

12.6	  Topmarks (see M4 - §463)	A.72

12.7	  Retro-reflectors	A.72

12.8	  Lights (see M4 - §470)	A.73

12.8.2	  Types and functions of lights (see M4 - §471.1)	A.73

12.8.3	  Rhythms of lights (see M4 - §471.2)	A.73

12.8.4	  Elevations of lights (see M4 - §471.6)	A.73

12.8.5	  Times of exhibition and exhibition conditions	A.73

12.8.5.1	Night lights	A.73

12.8.5.2	Unwatched lights (see M4 - §473.1)	A.74

12.8.5.3	Occasional lights (see M4 - §473.2)	A.74

12.8.5.4	Daytime lights (see M4 - §473.4)	A.74

12.8.5.5	Fog lights (see M4 - §473.5)	A.74

12.8.6	  Sector lights and lights not visible all round (see M4 - §475)	A.74

12.8.6.1		Sector lights (see M4 - §475.1)	A.74

12.8.6.2	Lights obscured by obstructions (see M4 - §475.3)	A.75

12.8.6.3	White fairway sectors (see M4 - §475.5)	A.75

12.8.6.4	Leading lights (see M4 - §475.6)	A.75

12.8.6.5	Directional lights (see M4 - §475.7)	A.76

12.8.6.6	Moiré effect lights (see M4 - §475.8)	A.76

12.8.7	  Various special types of lights	A.76

12.8.8 	 Light structures	A.76

12.9	  Radio stations (see M4 - §480 to §485)	A.77

12.9.1	  Marine and aero-marine radiobeacons (see M4 - §481)	A.77

12.9.2	  Aeronautical radiobeacons (see M4 - §482)	A.77

12.9.3	  Radio direction-finding stations (see M4 - §483)	A.77

12.9.4	  Coast radio stations providing QTG service (see M4 - §484)	A.77

12.10	 Radar beacons (see M4 - §486)	A.78

12.11	 Radar surveillance systems (see M4 - §487)	A.78

12.11.1	 Radar range (see M4 - §487.1)	A.78

12.11.2	 Radar reference lines (see M4 - §487.2)	A.78

12.11.3	 Radar station (see M4 - §487.3)	A.78

12.12	 Radar conspicuous objects (see M4 - §485.2)	A.79

12.13	 Radio reporting (calling-in) points (see M4 - §488)	A.79

13.	  Marine services and signal stations	A.79

13.1	  Pilot stations (see M4 - §491)	A.79

13.1.1	  Pilot stations ashore(see M4 - §491.3 and 491.4)	A.79

13.1.2	  Pilot boarding places (see M4 - §491.2)	A.80

13.2	  Coastguard stations (see M4 - §492)	A.80

13.3	  Rescue stations (see M4 - §493)	A.80

13.4	  Signal stations (see M4 - §494 to §497)	A.81

14.	  Geographic names	A.82

15.	  Collection objects	A.82

Figures

Figure 1 – Heights and elevations	 A.13

Figure 2 – Shoreline constructions	A.14

Figure 3 - Landmarks	A.30

Figure 4 – Depth contours	A.32

Figure 5 – Geometry of depth areas	A.34

Figure 6 – Depth areas	A.35

Figure 7 – Depth areas at seaward end of a non-navigable waterway and approaches	A.37

Figure 8 – Depth areas adjoining a shoreline construction	A.38

Figure 9 – Depth areas adjoining a dredged area	A.38

Figure 10 – Dredged areas adjoining dredged areas	A.39

Figure 11 – Linear depth areas along dredged area boundaries	A.40

Figure 12 – Area of wrecks	A.46

Figure 13- Seabed areas	A.48

Figure 14 – Mooring trot	A.51

Figure 15 – Navigation lines and recommended tracks	A.53

Figure 16 – Range systems	A.53

Figure 17 - Junction	A.54

Figure 18 – One-way traffic flow in a two-way route	A.58

Figure 19 – Maritime jurisdiction areas	A.60

Figure 20 – Navigation aids	A.68

Figure 21 – Buoyage system and direction	A.69

Figure 22 – Obscured light sectors	A.75

Page intentionally left blank

# 1.			Introduction

## 1.1		General

The following clauses specify the conventions that are to be used to encode the geometry and semantic description of each object in an ENC. This document is laid out, as far as possible along the lines of the IHO publication “Chart specifications of the IHO - M4".

This document describes how to encode information that the cartographer considers to be relevant to a specific navigational purpose. The content of the ENC is at the discretion of the producing authority provided that the conventions described below are followed.

This document is intended for use with Edition 2.0 of the ENC Product Specification (S-57 Appendix B1, Annex A) contained within S-57 Edition 3.1 which was adopted in November 2000.

For details of the use of this document by Hydrographic Offices still producing ENCs conforming to S-57 Edition 3.0, see the Introduction to the Explanatory Booklet which was issued with S-57 Edition 3.1.

## 1.2		Presentation of the document

The following conventions are used:

- Presentation conventions:	Object class:	AIRARE

Attribute:	VALSOU

Prohibited attribute:	VERDAT

Attribute value:	-2.4

- Attributes A: For each object class, the whole list of set A is given with the specific values for this feature object.

- Attributes B, attributes C: Except for attributes INFORM, NINFOM, and more rarely SORDAT, the attributes from sets B and C are not mentioned in the following lists; however,  this does not mean that their use is prohibited.

## 1.3		Use of language

Within this document "must" indicates a mandatory requirement.

# 2.			General rules

## 2.1		Cartographic framework

### 2.1.1		Horizontal datum

The horizontal datum must be unique in a data set and must be WGS 84. It is given in the “Horizontal Geodetic Datum” [HDAT] subfield of the “Data Set Parameter” [DSPM] field.

The use of meta object M_HDAT is prohibited.

The use of the attribute HORDAT for any spatial object is prohibited.

The meta object M_HOPA can be used to provide the shift from another horizontal datum to WGS 84 (see ENC Product Specification clause 4.1).

Where data has been transformed to WGS 84 from another datum, the HO may wish to indicate the accuracy of the transformation. This should be done using the attribute INFORM on the meta object M_QUAL.

### 2.1.2		Vertical datum

The vertical datum applies to the attributes ELEVAT, HEIGHT, VERCCL, VERCLR, VERCOP and VERCSA.

The default value for the entire data set is given in the “Vertical Datum” [VDAT] subfield of the “Data Set Parameter” [DSPM] field.

If the vertical datum is different to the value given in the VDAT subfield for some part of the data set, it  must be encoded as meta object M_VDAT. The areas covered by these meta objects must be mutually exclusive.

Meta object:		Vertical datum (M_VDAT)

Attributes:		VERDAT		INFORM		NINFOM

Height contours, going across areas having different values of vertical datum, must be divided into several objects at the border of these areas.

Various datums are used on paper charts and are used in the same way for ENC. For example different datums may be used for the following:

- altitude of spot heights, heights contours, landmarks,

- elevation of lights,

- vertical clearance.

Where different vertical datums are used for the various vertical measurements, the default value given by the subfield VDAT or a meta object M_VDAT applies to the first group of the previous list. Elevation of lights and vertical clearances refers to the VERDAT given in the individual object.

### 2.1.3		Sounding datum

The sounding datum must be constant over large areas. It applies to the attributes VALSOU, DRVAL1 and DRVAL2 and VALDCO.

The default value for the entire data set is given in the “Sounding Datum” [SDAT] subfield of the “Data Set Parameter” [DSPM] field.

If the sounding datum is different to the value given in the SDAT subfield for some part of the data set, it must be encoded as meta object M_SDAT. The areas covered by these meta objects must be mutually exclusive.

Meta object:		Sounding datum (M_SDAT)

Attributes:		VERDAT		INFORM		NINFOM

The use of VERDAT for individual objects related to depth is not allowed (object classes DEPARE, DEPCNT, DRGARE, OBSTRN, SOUNDG, UWTROC, WRECKS).

Depth contours and depth areas, going across areas having different values of sounding datum, must be divided into several objects at the border of these areas.

### 2.1.4		Units

The depth, height and positional accuracy units are unique in a data set. They must  be metres. They are given in the “Units of Depth Measurement” [DUNI],  “Units of Height Measurement” [HUNI] and “Units of Positional Accuracy” [PUNI] subfields of the “Data Set Parameter” [DSPM] field.

The use of meta object M_UNIT is prohibited.

### 2.1.5		Dates

When encoding dates using CPDATE, DATEND, DATSTA, PEREND, PERSTA, SORDAT, SUREND and SURSTA if no specific year, month or day is required, the following values should apply in conformance to ISO 8601:1988.

- No specific year required, same day each year:	--MMDD

- No specific year required, same month each year:	--MM

- No specific day required:	CCYYMM

- No specific month required:	CCYY

- Notes: CCYY = calendar year; MM = month; DD = day.

#### 2.1.5.1	 	Seasonal Objects

Seasonal Objects should be encoded using STATUS = 5 (periodic/intermittent). Where start and/or end dates of the season are known they should be encoded in PERSTA and PEREND.

### 2.1.6		Times

The attributes TIMSTA and TIMEND are used to encode the begining and end of an active period for an object. When using these attributes all times should be encoded as Coordinated Universal Time (UTC). ISO 8601 states that the format for UTC time should be CCYYMMDDThhmmssZ (where ‘T’ is a separator). The attribute definition for TIMEND and TIMSTA (in Chapter 2 of S-57 Appendix A) states that the mandatory format is CCYYMMDDThhmmss (ie. the ‘Z’ is missing). For Edition 3.1 of S-57 the format defined in Chapter 2 of S-57 Appendix A should be used and interpreted as UTC.

## 2.2		Data quality description

### 2.2.1		Production information and up-to-dateness

The producer of the ENC is indicated in the file name and in the “Producing Agency” [AGEN] subfield of the “Data Set Identification” [DSID] field. The use of the meta object M_PROD is prohibited.

### 2.2.2		Up-to-dateness information

Up-to-dateness information is given in the file name extension and in the “Edition Number” [EDTN], “Update Number” [UPDN] , “Update Application Date” [UADT] , “Issue Date” [ISDT]  subfields of the “Data Set Identification” [DSID] field.

### 2.2.3		Quality and accuracy of bathymetric data

Information about quality, reliability and accuracy of bathymetric data may be given by:

- the meta object M_QUAL for an assessment of the quality of bathymetric data,

- the meta object M_SREL for additional information about the survey,

- the attributes QUASOU, SOUACC and TECSOU on groups of soundings or individual objects.

- the attributes POSACC and QUAPOS on the spatial objects.

For the mariner M_QUAL generally provides the most useful information. So the use of the meta object M_QUAL is mandatory.

More detailed information is given by M_SREL. For example, in incompletely surveyed areas, lines of passage soundings may be indicated as such using a linear M_SREL object. This information is more difficult for the mariner to interpret. Therefore  the use of this meta object is optional.

For individual objects (wrecks, obstructions...) or small groups of soundings the attributes QUASOU, SOUACC and TECSOU may be used.

#### 2.2.3.1	 	Quality of bathymetric data

The meta object M_QUAL defines areas within which uniform assessment exists for the quality of bathymetric data and must be used to provide an assessment of the overall quality of bathymetric data to the mariner. Areas of a cell containing depth data or bathymetry must be covered by meta objects M_QUAL. These meta objects are mutually exclusive.

Meta object:		Quality of data (M_QUAL)

Attributes:		CATQUA		CATZOC		DRVAL1

DRVAL2 - 	the maximum depth to which the quality information applies

POSACC		SOUACC	SURSTA		SUREND		TECSOU		VERDAT

INFORM		NINFOM

Remarks:

- The attributes POSACC, SOUACC and TECSOU may be used to upgrade the components that are of higher accuracy.

- A CATZOC category indicates that the soundings within M_QUAL meet the minimum criteria described in the CATZOC definition table. A CATZOC category may be further sub-divided by specifying depth and positional accuracies using the POSACC and SOUACC attributes.

- DRVAL1 must only be used within M_QUAL when a swept area occupies the entire M_QUAL.

- DRVAL2 is used to specify the maximum depth to which the CATZOC category applies. When DRVAL2 is specified, the CATZOC category applies to soundings equal to or shoaler than DRVAL2. No quality information is provided for soundings deeper than DRVAL2.

- POSACC may only be used to specify a higher positional accuracy of the soundings than the CATZOC category indicates. When DRVAL1 is specified POSACC must not be used. There is no positional accuracy information provided for any underlying soundings.

- SOUACC may only be used to specify a higher depth accuracy of the soundings than the CATZOC category indicates. When DRVAL1 is specified, SOUACC refers only to the accuracy of the swept depth defined by DRVAL1. There is no depth accuracy information provided for any underlying soundings.

- When the M_QUAL comprises soundings of 2 or more different techniques, the attribute TECSOU should not be used.

- When the M_QUAL comprises data for only one survey, the date of survey is specified using the attribute SUREND. When the M_QUAL comprises data from 2 or more surveys the date of the oldest survey is specified using SURSTA and the date of the most recent survey specified using SUREND.

- Additional quality information may be given by the meta object M_SREL.

- For M_QUAL areas over land CATZOC should be set to 6 (unassessed).

#### 2.2.3.2	 	Survey reliability

The survey reliability is encoded by means of the meta object M_SREL. The use of this meta object is optional.

Meta object:		Survey reliability (M_SREL)

Attributes:		QUAPOS	QUASOU	SCVAL1		SCVAL2		SDISMN		SDISMX		SURATH

SUREND		SURSTA		SURTYP		TECSOU		INFORM		NINFOM

Remarks:

- SOUACC and TECSOU are encoded in M_QUAL.

- The attribute SURATH is used to indicate the survey source, see clause 2.2.5.1.

#### 2.2.3.3	 	Quality of sounding

- The quality of sounding is encoded with QUASOU, when different to the meta object M_SREL.

#### 2.2.3.4	 	Sounding accuracy

- The accuracy of sounding is encoded with SOUACC, when different to the meta object M_QUAL.

#### 2.2.3.5	 	Technique of sounding measurement

The technique of sounding measurement is encoded using TECSOU, when different to the meta object M_QUAL.

### 2.2.4		Accuracy

#### 2.2.4.1	 	Quality of positions

The meta object M_ACCY may be used to provide an overall accuracy of position for all non bathymetric features. It must not be used to provide the accuracy of bathymetric information.

Meta object:		Accuracy of data (M_ACCY)

Attributes:		HORACC	POSACC 	SOUACC	VERACC		INFORM		NINFOM

The attributes QUAPOS and POSACC may apply to the spatial object of any geo object if they are different to the relevant meta object.

QUAPOS gives qualitative information, whereas POSACC gives quantitative information.

#### 2.2.4.2	 	Horizontal accuracy

The accuracy of a horizontal clearance (HORCLR) is given by the attribute HORACC.

The attribute HORACC only applies to HORCLR. There is no attribute to express the accuracy of HORLEN and HORWID.

#### 2.2.4.3	 	Vertical accuracy

The accuracy of vertical clearances (VERCLR, VERCOP, VERCSA, VERCCL) is given by the attribute VERACC.

If several vertical clearances are given for one object, the accuracy given is that of the least accurate.

### 2.2.5		Source of data

#### 2.2.5.1	  	Source of bathymetric data

Details of the source surveys used in compilation are encoded using the meta object class M_SREL described in clause 2.2.3.2.

Remarks:

- The survey authority is encoded in SURATH and not SORIND.

- If a point object (sounding, wreck ...) has a source different to that given by M_SREL, this other source may be encoded by the attributes SORIND and SORDAT of the object, if this information is considered to be useful for the mariner.

#### 2.2.5.2	  	Source of other data

If it is considered to be of use to the mariner, the source of non-bathymetric information, may be encoded in the attributes SORIND and SORDAT of the individual objects.

### 2.2.6		Compilation scale

The default value for the entire data set is given in the “Compilation Scale of Data” [CSCL] subfield of the “Data Set Parameter” [DSPM] field. The default value should be the compilation scale appropriate to the greater part of the data in the cell.

If the compilation scale is different to the value given in the CSCL subfield for some part of the data set, it must be encoded using the meta object M_CSCL. The areas covered by these M_CSCL meta objects are mutually exclusive.

Meta object:		Compilation scale of data (M_CSCL)

Attributes:		CSCALE		INFORM		NINFOM

Remarks:

- The compilation scale provides the reference value for the overscale indication.

### 2.2.7		Use of the attribute SCAMIN

The SCAMIN of an object determines the display scale below which the object must no longer be displayed. Its purpose is to reduce clutter, to prioritise the display of objects and to improve display speed. In encoding its value, the producer should consider the factors above as well as the scale at which the object is no longer likely to be required for navigation.

In order to optimize the performance and clarity of the ENC it is strongly recommended that SCAMIN is used.

Remarks:

- Note that SCAMIN only affects the display of an object, not its presence in the SENC.

- If SCAMIN is missing the object must be displayed at all scales.

- Where SCAMIN is used it must always be set to a scale less (ie. to a smaller scale) than the compilation scale of the data. Failure to follow this rule would mean that objects would not be drawn on screen until the overscale warning is displayed.

- Group1 objects must always be displayed. Therefore, SCAMIN must not be used with Group1 objects.

- If the same object exists in cells of different navigational puposes, the same SCAMIN value should be assigned to each occurence of the object.

## 2.3		Textual information

The attributes INFORM, NINFOM, TXTDSC and NTXTDS must not be used when it is possible to encode the information by means of any other attribute.

INFORM and NINFOM contain information as text, whereas TXTDSC and NTXTDS encode the name of an external file.

The text contained in INFORM and NINFOM is ASCII text. Formatting characters (C0 as defined in S-57 Part 3, Annex B) are prohibited. Therefore, INFORM and NINFOM are generally used for short notes (e.g. caution notes from paper charts), or to transfer information which cannot be encoded by other attributes or to give more detailed information.

The files referenced by TXTDSC and NTXTDS can contain formatted texts. Text files that form part of the ENC must be ASCII files. They may contain formatting characters. Therefore, the files referenced by

TXTDSC and NTXTDS are generally used for longer texts (e.g. longer chart notes, tables or paragraphs from sailing directions). It is up to the encoder to determine the most suitable means of encoding a particular piece of text.

## 2.4		Colours and colour patterns

When an object has multiple colours they are to be interpreted as follows:

- for horizontal stripes(COLPAT = 1) the first colour in the list is the top-most and subsequent colours follow sequentially from top to bottom.

- for vertical stripes (COLPAT =2) the first colour in the list is the left-most and subsequent colours follow sequentially in left to right order.

- for diagonal stripes(COLPAT = 3) the first colour in the list is the top-left-most and subsequent colours follow sequentially from top left to bottom right.

- for squares (COLPAT = 4) the first colour in the list is the top-left-most square and subsequent colours follow sequentially from left to right along the top row then to the left-most square on the next line and so on until the bottom right-most square is reached.

- for border stripes (COLPAT = 6) the first colour in the list is the border stripe, the second colour is that of the background. Where a border stripe is combined with other patterns the border stripe will be the first colour in the list, subsequent colours must be interpreted in accordance with the rules defined for the additional patterns. Therefore, if a pattern contains a border stripe as well as other patterns, the border stripe must be the first value in the list of COLPAT.

## 2.5		Reference to other publications

Reference to other nautical publications is made by means of the meta object class M_NPUB.

Meta object:		Nautical publication information (M_NPUB)

Attributes:		INFORM		NINFOM		PUBREF

## 2.6 		Updating

### 2.6.1		Issuing updates in advance

Under certain conditions it might be necessary for a data producer to issue update information in advance. For example, a change in a traffic routeing system must be made public before the new situation is implemented. Within an ER data set there is no means of indicating the date at which an update must be applied. Therefore, when an ER data set is received by an end user it must be applied immediately. To avoid situations where update information would cause target data to reflect a situation which does not yet exist, the following coding rules must be followed:

a) 	If the future update involves the addition of objects to the existing data (e.g. a new lighthouse), the  attribute DATSTA on any new object  must contain the date at which the update becomes active.

b) If the future update involves the modification of existing objects (e.g. a change in a traffic routeing system), this must be treated as a deletion of the existing objects and replacement with new objects. See a) above and c) below.

c) If the future update involves the deletion of existing objects (e.g. the removal of a buoy), the update message should set the DATEND attribute of all objects to be deleted to the date at which the update

becomes active. NB. this update message does not actually delete the objects from the data set,  it simply indicates that on the date held in DATEND they become obsolete. A further update to actually delete the obsolete objects from the data set should be sent at the time that the change in the real world occurs.

d) To highlight to the mariner that an  update will take place in the future, it is recommended that a caution area (CTNARE) is created covering the location at which  the future update will take place. A warning note specifying, in plain language, the nature of the future change should be encoded either in INFORM or in a file referenced by the TXTDSC attribute of the CTNARE. The DATEND attribute of the CTNARE must be set to the date at which the update becomes active.

Note that changes to DATSTA and DATEND cannot be applied to spatial objects. Therefore, a change to geometry of a real world object  (e.g. the relocation of a buoy) to be applied in the future can only be achieved by updating both the feature and spatial objects involved.

As a consequence of issuing future updates, more than one instance of a real world object could exist in the data.

## 2.7		Multiple objects

On some sources the presence of multiple object in close proximity is generalised to a single object with a text string indicating the presence of the other objects. In such cases one object of the appropriate class should be encoded and the true number of objects should be encoded in INFORM (eg ‘3 chimneys’). If the true number of objects is not known the text ‘more than one’ should be encoded in INFORM.

  - Minimal depiction areas (see M4 - §404)

Where minimal depiction areas exist within a specified ENC navigational purpose, they should be encoded using one of the following options:

2.8.1	Wide blank areas

Meta object:		M_COVR

Attribute:			CATCOV = 2 (no coverage available)

    - Simplified or minimal depiction areas

- Bathymetry is encoded as described in clause 5.8.3.

- The information that does not relate to bathymetry but is relevant to group 1 object (islands, shoreline constructions, floating docks, hulkes…) may be encoded.

- One CTNARE covering the whole area should be created. INFORM or TXTDC may be encoded using one of the following options:

Where larger scale coverage is available:

Most features are omitted in this area, including bathymetry. The minimal depiction of detail in this area does not support safe navigation, therefore mariners should use a more appropriate navigational purpose.

Any other information contained in chart notes should be incorporated within, or replace completely the above statement.

Where no larger scale coverage is available:

Most features are omitted in this area, including bathymetry. The minimal depiction of detail in this area does not support safe navigation.

This statement should be supplemented by additional cautionary information from chart notes relating to any authority to be consulted before navigating in the area.

# 3.			Time Varying Objects

## 3.1		Magnetic data (see M4 - §270)

### 3.1.1		Magnetic variation

Until a world magnetic model is universally available for inclusion in ECDIS, magnetic variation should be encoded using the object class MAGVAR. As a minimum, updates should be supplied to coincide with changes of epoch (i.e. every five years).

Geo object:		Magnetic variation (MAGVAR)

Attributes:		DATEND		DATSTA		RYRMGV	VALACM		VALMAG		INFORM		NINFOM

### 3.1.2		Abnormal magnetic variation (see M4 - §274)

Geo object:		Local magnetic anomaly (LOCMAG)

Attributes:		NOBJNM		OBJNAM		VALLMA		INFORM		NINFOM

- If the area cannot be defined, the feature can be represented as a point object.

## 3.2		Tidal data (see M4 - §406)

The inclusion of tidal information in ENC data sets is optional.

For Standard Ports the appropriate national Hydrographic Office or an organization authorized by it should provide the predictions.

For Secondary Ports, the appropriate Hydrographic Office or an organization authorized by it should, where possible, provide the predictions. Should such predictions not be available, the ECDIS manufacturer should approach the appropriate national HO for advice regarding the best methods of prediction for the Secondary Ports in their area of responsibility.

It is recommended that each appropriate Hydrographic Office, or an organization authorized by it, should determine the spatial limits for applying tidal information where applicable and the number of tidal stations

to be used in modelling. The HOs should be responsible for determining the best methodologies to be used in their areas of responsibility.

The accuracy of tidal information is to be assessed to the confidence level of 95% irrespective of the method of application or its source.

### 3.2.1		Time series data

Times and heights of high and low waters may be encoded using the object class T_TIMS. In addition, where the data is available, a regular time series of tidal heights may also be encoded using this object class.

Geo object:		Tide-time series (T_TIMS)

Attributes:		NOBJNM		OBJNAM

TIMEND		TIMSTA -	specify the period for which the time series is valid

T_ACWL		T_TSVL		T_TINT		T_HWLW	STATUS		INFORM		NINFOM

### 3.2.2		Prediction by harmonic methods

Parameters for the prediction of tidal heights using harmonic methods may be supplied using the object-class T_HMON. The supplier of parameters must be consulted on how to use this data and which calculation algorithms to use with the data.

Geo object:		Tide-harmonic prediction (T_HMON)

Attributes:		NOBJNM		OBJNAM		T_ACWL

T_MTOD -	1 - simplified harmonic method of tidal prediction

2 - full harmonic method of tidal prediction

T_VAHC		STATUS		INFORM		NINFOM

### 3.2.3		Prediction by non-harmonic methods

Parameters for the prediction of tidal heights using time and height differences may be encoded using the object-class T_NHMN.

The reference port to be used for these predictions must be identified using a collection object (C_ASSO) between the tidal parameter object of the reference port (T_TIMS or T_HMON) and the object T_NHMN of the secondary port. If the reference port is not located within the data set or exchange set, then its tidal parameters may be supplied as a geo object with no geometry.

Other non-harmonic methods for predicting tidal heights are not supported at this time.*

Geo object:		Tide-non-harmonic prediction (T_NHMN)

Attributes:		NOBJNM		OBJNAM		T_ACWL		T_MTOD		T_THDF		STATUS		INFORM

NINFOM

3.3		Tidal stream data (see M4 - §407)

### 3.3.1		Tidal stream (flood/ebb)

When tidal stream information is limited to only flood and ebb directions and/or values, it may be encoded using the object class TS_FEB.

Geo object:		Tidal stream-flood/ebb (TS_FEB)

Attributes:		CAT_TS	-	1 - flood stream

2 - ebb stream

3 - other tidal flow

CURVEL -	maximum rate (during springs)

DATEND		DATSTA		NOBJNM		OBJNAM		ORIENT		PEREND 	PERSTA

INFORM		NINFOM

### 3.3.2		Tidal stream time series

- Time series data for tidal streams may be encoded using the object class TS_TIS.

Geo object:		Tidal stream - time series (TS_TIS)

Attributes:		NOBJNM		OBJNAM

TIMEND		TIMSTA -	specify the period for which the time series is valid

TS_TSV		T_TINT		STATUS		INFORM		NINFOM

### 3.3.3		Prediction by harmonic methods

Parameters for the prediction of tidal streams using harmonic methods may be supplied using the object class TS_PRH. The supplier of parameters must be consulted on how to use this data and which calculation algorithms to use with the data.

Geo object:		Tidal steam - harmonic prediction (TS_PRH)

Attributes:		NOBJNM		OBJNAM

T_MTOD -	1 - simplified harmonic method of tidal prediction

2 - full harmonic method of tidal prediction

T_VAHC		STATUS		INFORM		NINFOM

### 3.3.4		Prediction by non-harmonic methods

Parameters for the prediction of tidal streams using time and rate differences may be encoded using the object-class TS_PNH.

The reference station to be used for these predictions must be identified using a collection object (C_ASSO) between the stream parameter object of the reference station (TS_TIS or TS_PRH) and the object TS_PNH of the secondary station. If the reference station is not located within the data set or exchange set, then its tidal stream parameters may be supplied as a geo object with no geometry.

Other non-harmonic methods for predicting tidal stream are not supported at this time.

Geo object:		Tidal steam - non-harmonic prediction (TS_PNH)

Attributes:		NOBJNM		OBJNAM		T_THDF		T_MTOD		STATUS		INFORM		NINFOM

### 3.3.5		Tidal stream panels

The information generally shown on paper charts as a tidal stream panel and stations may be encoded using the object class TS_PAD.

Tidal stream values encoded in this way should be mean spring rates, i.e. the tidal stream rates associated with a tidal range which is defined as the difference in height between MHWS and MLWS.

Geo object:		Tidal steam panel data (TS_PAD)

Attributes:		NOBJNM		OBJNAM		TS_TSP		INFORM		NINFOM

## 3.4		Current data (see M4 - §408)

- Non-gravitational currents may be encoded using the object class CURENT.

Geo object:		Current (CURENT)

Attributes:		CURVEL		DATEND		DATSTA		NOBJNM		OBJNAM		ORIENT		PEREND

PERSTA		INFORM		NINFOM

# 4.			Topography

## 4.1		Land area

Mangrove areas or land areas that are never covered by the sea must be encoded as LNDARE.

Rivers, canals, lakes, basins and docks which are not navigable at compilation scale must be encoded on top of a LNDARE.

- Area LNDARE are always part of Group 1.

Geo object:		Land area (LNDARE)

Attributes:		CONDTN	OBJNAM		NOBJNM		INFORM		NINFOM		STATUS

Remarks:

- The category of landscape is described by LNDRGN (see clause 4.7.1).

- A LNDARE is usually an area object; it may however be sometimes of type point (islet, rock which does not cover,...) Or of type line (islet, offshore bar, isthmus, ...).

## 4.2		Vertical measurements (see M4 - §302 and §303)

### 4.2.1		Vertical datum

- See clause 2.1.2.

### 4.2.2		Heights and elevations

For natural features (hills, coastlines, slopes, etc...) the altitude is encoded by the attribute ELEVAT  (figure 1 - a).

For artificial features (landmarks, buildings...) or trees:

- the altitude of the ground level at the base of the object is encoded by the attribute ELEVAT (b),

- the altitude of the highest point of the object is encoded by the attribute HEIGHT (c),

- the height of the object above ground level is encoded by the attribute VERLEN (d).

Figure 1 – Heights and elevations

## 4.3		Control points (see M4 - §304 to §306)

Geo object:		Control point (CTRPNT)

Attributes:		CATCTR		DATEND		DATSTA		ELEVAT		NOBJNM		OBJNAM		VERACC

VERDAT		INFORM		NINFOM

## 4.4		Distance marks (see M4 - §307)

Geo object:		Distance mark (DISMAR)

Attributes:		CATDIS		DATEND		DATSTA		NOBJNM		OBJNAM

INFORM - 	the value of distance with its unit of measurement

###### NINFOM

## 4.5		Coastline

In all cases the coastline is encoded as either a COALNE (line) or a SLCONS (line or area). These features form the border of the land area (LNDARE).

Sections of the natural coastlines, lake shores and river banks should be encoded as COALNE whereas sections of the artificial coastlines, lake shores, river banks, canal banks and basin borders should be encoded as SLCONS. The exception to this general rule is when a lake, river, canal, or basin is not navigable at the compilation scale, in which case, the borders are not encoded.

### 4.5.1		Natural coastline (see M4 - §312)

Geo object:		Coastline (COALNE)

Attributes:		CATCOA		COLOUR	CONRAD	CONVIS		ELEVAT		NOBJNM		OBJNAM

VERACC		VERDAT		INFORM		NINFOM

Spatial objects associated with unsurveyed coastlines (see M4 - §311) are encoded with QUAPOS = 2.

The attribute CATCOA encodes the nature of the coastline itself. Other area objects may be used to describe the land region near the coastline (see clause 4.7).

### 4.5.2		Artificial coastline (see M4 - §313 and §320)

Geo object:		Shoreline construction (SLCONS)

##### Attributes:		CATSLC		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND

DATSTA		HEIGHT

HORACC -	applies to HORCLR

HORCLR		HORLEN		HORWID		NATCON		NOBJNM		OBJNAM		STATUS

VERACC		VERDAT		VERLEN		WATLEV		INFORM		NINFOM

Figure 2 represents a shoreline construction such as a mole with a berthing facility (INT1 - IF12), with a relatively flat top (abcdlmna), with sloping sides partly above high water (nmldefgn) and partly intertidal (dopqrhgfed).

Figure 2 – Shoreline constructions

Remarks:

- Each of the three parts of the shoreline construction may be encoded as separate SLCONS (as areas) or alternatively, the boundaries of the shoreline construction may be encoded with SLCONS (as lines).  If part of the SLCONS boundary has a different characteristic (eg (bc) CATSLC= 6 or 15), it should be encoded as a seperate line (SLCONS). If SLCONS is encoded as an area, the masked line (an) is optional.

- In this example, SLCONS above the high water line must also be encoded with LNDARE (as an area) and the intertidal SLCONS must also be encoded with DEPARE (as an area).

- SLCONS objects should be broken into their constituent parts and attributed using CATSLC as indicated on the source. If the presence of an object is indicated by a textual reference only on the source, without a clear symbol (eg ‘pier’, ‘groynes’, ‘posts’ etc.), it should be encoded as a CTNARE with the textual reference encoded in the attribute INFORM.

- Intertidal or submerged artificial rock walls such as training walls, that are not attached to the shoreline are to be encoded in the following manner CATSLC = 7 (training wall) with WATLEV = 3 (always under water/submerged) or WATLEV = 4 (covers and uncovers).

## 4.6		Harbour installations

### 4.6.1		Harbour facilities

Geo object:		Harbour facility (HRBFAC)

Attributes:		CATHAF		CONDTN	DATEND		DATSTA		NATCON		NOBJNM		OBJNAM

PEREND		PERSTA		STATUS		INFORM		NINFOM

Remarks:

- Depending on the navigational purpose, the harbour facility will be defined by: an area including docks, basins, and dockside equipment or a point as shown by a symbol (INT1 - IF 10, IU 1.1) on paper charts.

### 4.6.2		Berths (see M4 - §323)

Geo object:		Berth (BERTHS)

Attributes:		DATEND		DATSTA

DRVAL1 -	minimum depth at berth

NOBJNM		OBJNAM -	name or number of the berth

PEREND		PERSTA		QUASOU	SOUACC	STATUS		VERDAT

INFORM -	maximum draft permitted at berth

NINFOM

Remarks:

- The berth encodes the named place at a wharf. The wharf itself is encoded as a shoreline construction (see clause 4.5.2).

- Landing places for boats are encoded as small craft facilities (see clause 4.6.5).

### 4.6.3		Harbour offices (see M4 - §325)

Harbour offices are encoded as BUISGL with FUNCTN -	2	- harbour-master’s office

- 3	- custom office

- 4	- health office

11	- pilot office

### 4.6.4		Checkpoints

An official place to register, declare or check goods and  people is encoded as:

Geo object:		Checkpoint (CHKPNT)

Attributes:		CATCHP		NOBJNM		OBJNAM		STATUS		INFORM

NINFOM

Remarks:

- This object only encodes the function. In addition the physical object (building, facilities...) may be encoded.

### 4.6.5 		Small craft facilities

Geo object:		Small craft facility (SMCFAC)

Attributes:		CATSCF		NOBJNM		OBJNAM		PEREND		PERSTA		STATUS		INFORM

NINFOM

Remarks:

- This object encodes only the function. In addition the physical object (building, stairs...) may be encoded.

### 4.6.6		Docks

#### 4.6.6.1	 	Dry docks (see M4 - §326.1)

Geo object:		Dry dock (DRYDOC)

Attributes:		CONDTN

DRVAL1 -	minimum depth in the dock when the gate is open.

HORACC -	applies to HORCLR

HORCLR		HORLEN		HORWID		NOBJNM		OBJNAM		QUASOU 	SOUACC

STATUS		VERDAT

INFORM -	maximum draft permitted

NINFOM

Remarks:

- A dry dock is also covered with a LNDARE. The boundary of a dry dock is not encoded by a separate object.

#### 4.6.6.2	 	Floating docks (see M4 - §326.2)

Geo object:		Floating dock (FLODOC)

Attributes:		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND		DATSTA

DRVAL1

HORACC -	applies to HORCLR

HORCLR		HORLEN		HORWID		LIFCAP		NOBJNM 	OBJNAM		STATUS

VERACC		VERDAT		VERLEN

INFORM -	maximum draft permitted

NINFOM

Remarks:

- Area FLODOC are part of Group 1.

#### 4.6.6.3	 	Tidal and non-tidal basins (see M4 - §326.3 and §326.4)

Geo object:		Dock area (DOCARE)

Attributes:		CATDOC -	1 - tidal

2 - non tidal (wet dock)

CONDTN	DATEND		DATSTA

HORACC -	applies to HORCLR

HORCLR -	size of the entrance

NOBJNM		OBJNAM		STATUS		INFORM		NINFOM

Remarks:

- If the dock is navigable at compilation scale it must be encoded as DEPARE or DRGARE (see clause 5.4.8) and the geo objects making up the dock limits must be encoded as the respective object classes such as COALNE, SLCONS, GATCON etc. If the name of the dock is required it must be encoded as a SEAARE. It must not also be encoded as a DOCARE object.

- If the dock is not navigable at compilation scale it must be encoded as a DOCARE with a LNDARE or UNSARE underneath. The name of the dock may be encoded using OBJNAM. The boundary of a dock is not encoded by a separate object.

- In a non-tidal basin, depths may refer to a sounding datum different to that in open waters. If this area is navigable at the compilation scale, the value of this datum is carried by a meta object M_SDAT with VERDAT = 24 (local datum).

- Smaller dock areas may be included in major dock areas, with different names or characteristics. To encode this fact, dock areas (DOCARE) and/or sea areas (SEAARE) may overlap.

#### 4.6.6.4	 	Gates (see M4 - §326.5 to §326.7)

- The object class GATCON is used to encode gates that control the flow of water.

Geo object:		Gate (GATCON)

Attributes:		CATGAT		CONDTN

DRVAL1 -	minimum depth over the sill

HORACC -	applies to HORCLR

HORCLR		NATCON		NOBJNM		OBJNAM		QUASOU	SOUACC	STATUS

VERACC -	applies to VERCLR

VERCLR

VERDAT -	applies to VERCLR (not DRVAL1)

INFORM		NINFOM

#### 4.6.6.5	 	Locks

Geo object:		Lock basin (LOKBSN)

Attributes:		DATEND		DATSTA

HORACC -	applies to HORCLR

HORCLR		HORLEN		HORWID		NOBJNM		OBJNAM		STATUS		INFORM

NINFOM

Remarks:

- If the lock is navigable at compilation scale it must be encoded as DEPARE or DRGARE (see clause 5.4.8) and the geo objects making up the limits of the lock must be encoded as the respective object classes such as COALNE,  SLCONS or GATCON.  If the name of the lock is required it must be encoded as a SEAARE. It must not also be encoded as a LOKBSN object.

- If the lock is not navigable at compilation scale it must be encoded as a LOKBSN with a LNDARE or UNSARE underneath. The name of the lock may be encoded using OBJNAM.

- The gates are encoded as GATCON with CATGAT = 4 (lock gate), 3 (caisson) or 6 (sluice). At small scale a lock may be encoded only by the gates, without a basin.

#### 4.6.6.6	 	Gridiron (see M4 - §326.8)

Geo object:		Gridiron (GRIDRN)

Attributes:		HORACC	HORLEN		HORWID		NATCON		NOBJNM		OBJNAM		STATUS

VERACC		VERLEN		WATLEV		INFORM		NINFOM

### 4.6.7		Mooring / warping facilities and pontoons

#### 4.6.7.1	 	Mooring / warping facilities (see M4 - §327)

Geo object:		Mooring / warping facility (MORFAC)

Attributes:		BOYSHP -	used only if CATMOR = 7

CATMOR -	1 - dolphin

2 - deviation dolphin

3 - bollard

4 - tie-up wall

5 - post or pile

6 - chain/wire/cable

7 - mooring buoy

COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND		DATSTA

HEIGHT		NATCON		NOBJNM		OBJNAM		PEREND		PERSTA		STATUS

VERACC		VERDAT		VERLEN		WATLEV		INFORM		NINFOM

Remarks:

- Stumps of posts or piles dangerous to navigation are encoded as OBSTRN with CATOBS = 1. If posts or piles are not dangerous they are encoded as MORFAC with CATMOR = 5 and CONDTN = 2.

- A pile is encoded as MORFAC with CATMOR = 5 when it has been identified as a mooring post, otherwise it is encoded as PILPNT.

- Area MORFAC must be covered by an area LNDARE.

#### 4.6.7.2	 	Piles

A pile that is not used as a mooring/warping facility is encoded as:

Geo object:		Pile (PILPNT)

Attributes:		CATPLE		COLOUR	COLPAT		CONDTN	CONVIS		DATEND		DATSTA

HEIGHT		NOBJNM		OBJNAM		VERACC		VERDAT		VERLEN

INFORM		NINFOM

Remarks:

- Stumps of piles that are dangerous to navigation are encoded as OBSTRN with CATOBS = 1.

#### 4.6.7.3	 	Pontoons (see M4 - §326.9)

Geo object:		Pontoon (PONTON)

Attributes:		CONDTN	CONRAD	CONVIS		DATEND		DATSTA		NATCON		NOBJNM

OBJNAM		PEREND		PERSTA		STATUS		VERACC		VERLEN		INFORM

NINFOM

Remarks:

- Area PONTON are part of Group 1.

### 4.6.8		Hulks

- A permanently moored ship is encoded as a HULKES.

Geo object:		Hulk (HULKES)

Attributes:		CATHLK		COLOUR	COLPAT		CONDTN 	CONRAD	CONVIS		HORACC

HORLEN		HORWID		NOBJNM		OBJNAM		VERACC		VERLEN		INFORM

NINFOM

Remarks:

- HULKES of type area are part of Group 1.

- Floating breakwaters are encoded as HULKES with CATHLK = 5.

### 4.6.9		Dockside buildings and structures

#### 4.6.9.1	 	Transit sheds and warehouses (see M4 - §328.1)

Transit sheds and warehouses are encoded as BUISGL (see clause 4.8.15) with FUNCTN = 15 and OBJNAM = name or number of the shed.

#### 4.6.9.2	 	Timber yards (see M4 - §328.2)

- Timber yards are encoded as PRDARE (see clause 4.8.13) with CATPRA = 6.

#### 4.6.9.3	 	Cranes (see M4 - §328.3)

Geo object:		Crane (CRANES)

Attributes:		CATCRN		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		HEIGHT

LIFCAP		NOBJNM		OBJNAM

ORIENT -	angular distance from the true north to the axis of the crane’s jib (generally perpendicular to the wharf)

###### RADIUS		STATUS

VERACC -	applies to VERCLR (not HEIGHT)

- VERCLR

VERDAT -	applies to VERCLR (not HEIGHT)

###### VERLEN		INFORM		NINFOM

Remarks:

- The position of a sheerleg or a travelling crane is defined as its resting position. The track is encoded as a RAILWY (see clause 4.8.2).

### 4.6.10	 	Works in progress and projected (see M4 - §329)

Features which are under construction or projected should be encoded in the same way as existing features (LNDARE, SLCONS, DRYDOC, LOKBSN, PIPSOL,...) with:

CONDTN -	1 - under construction

3 - under reclamation

5 - planned construction

SORDAT -	year or date of the information

The coastline existing before the beginning of works, remains encoded as COALNE or SLCONS until the completion of the works.

## 4.7		Natural features (see M4 - §350)

### 4.7.1	 	Natural sceneries

If there is a need to describe the natural scenery of the land or to give the geographic name of an area, land regions may be encoded.

Geo object:		Land region (LNDRGN)

Attributes:		CATLND		NATQUA		NATSUR		NOBJNM		OBJNAM		WATLEV		INFORM

NINFOM

Remarks:

- This object class has a use similar to SEAARE (see clause 8) but for the land.

- The area should be limited, if possible, by existing lines used by other objects (COALNE…). If necessary they may be limited by other lines created to close the area or to describe a new area.

- For capes, points, peninsulas… there is no specific value for CATLND, therefore the generic term "cape", "point", "peninsula"… may be included in OBJNAM and NOBJNM.

- Areas of land regions may overlap.

### 4.7.2		Height contours, spot heights (see M4 - §351)

Geo object:		Land elevation (LNDELV)

Attributes:		CONVIS		ELEVAT		NOBJNM		OBJNAM		VERACC		VERDAT		INFORM

NINFOM

Height contours are associated with line spatial objects, whereas spot heights are associated with point spatial objects.

Spatial objects associated with approximate contours or spot heights are encoded with QUAPOS = 4.

### 4.7.3		Marsh (see M4 - §312.2)

If the seaward edge of a marsh is coincident with the coastline it should be encoded as a COALNE with CATCOA = 8, and the spatial object may have the attribute QUAPOS = 4 (approximate) for the visible coastline.

The marsh area is encoded as a LNDRGN with CATLND = 2.

### 4.7.4		Dunes, sand hills (see M4 - §312.3)

Geo object:		Sloping ground (SLOGRD)

Attributes:		CATSLO -	3 - dune

4 - hill

COLOUR	CONRAD	CONVIS		NATCON		NATQUA		NATSUR		NOBJNM

OBJNAM		INFORM		NINFOM

If these features are positioned along the coastline, an object COALNE is also created.

To encode the height of one of these features, an object LNDELV is created.

### 4.7.5		Cliffs (see M4 - §312.1)

- Cliffs are encoded as SLOGRD and / or SLOTOP. (See also Remark)

Geo object:		Sloping ground (SLOGRD)

Attributes:		CATSLO -	6 - cliff

COLOUR	CONRAD	CONVIS		NATCON		NATQUA		NATSUR		NOBJNM

OBJNAM		INFORM		NINFOM

Geo object:		Slope topline (SLOTOP)

Attributes:		CATSLO -	6 - cliff

COLOUR	CONRAD	CONVIS		ELEVAT		NATCON		NATQUA		NATSUR

NOBJNM		OBJNAM		VERACC		VERDAT		INFORM		NINFOM

For example:

The object SLOGRD is used at large scale to indicate the horizontal extent of the cliff.

The object SLOTOP is used on its own to encode cliffs at small scale or in conjunction with SLOGRD to indicate the crest line of the cliff:

- when it is useful to know its elevation,

- to encode a cliff on land distant from the coastline,

Remarks:

- When the cliff is coincident with the coastline, the object COALNE, with CATCOA = 1 must be used.

### 4.7.6		Rivers (see M4 - §353)

Geo object:		River (RIVERS)

##### Attributes:		NOBJNM		OBJNAM

STATUS - 	5 - intermittent

###### INFORM		NINFOM

Remarks:

- If the river is navigable at compilation scale it must be encoded as DEPARE or DRGARE (see clause 5.4.8) and the river banks must be encoded as COALNE or SLCONS. If the name of the river is required it must be encoded as a SEAARE with CATSEA = 53. It must not also be encoded as a RIVERS object.

- If the river is not navigable at compilation scale it must be encoded as a RIVERS with a LNDARE or UNSARE underneath. The name of the river may be encoded using OBJNAM.

- Use of the object class RIVBNK is prohibited.

### 4.7.7		Rapids, waterfalls (see M4 - §353.5)

#### 4.7.7.1	 	Rapids

Geo object:		Rapids (RAPIDS)

Attributes:		NOBJNM		OBJNAM		VERACC		VERLEN		INFORM		NINFOM

- The area covered by rapids is also covered by an object RIVERS and an object LNDARE.

#### 4.7.7.2	 	Waterfalls

Geo object:		Waterfall (WATFAL)

Attributes:		CONVIS		NOBJNM		OBJNAM		VERACC		VERLEN		INFORM		NINFOM

### 4.7.8		Lakes (see M4 - §353.6)

Geo object:		Lake (LAKARE)

Attributes:		ELEVAT		NOBJNM		OBJNAM		VERACC		VERDAT		INFORM		NINFOM

Remarks:

- If the lake is navigable at compilation scale it must be encoded as DEPARE or DRGARE (see clause 5.4.8) and the lake shore must be encoded as COALNE or SLCONS. If the name of the lake is required it must be encoded as a SEAARE with CATSEA = 52. It must not also be encoded as a LAKARE object.

- If the lake is not navigable at compilation scale it must be encoded as a LAKARE with a LNDARE or UNSARE underneath. The name of the lake may be encoded using OBJNAM.

- Use of the object class LAKSHR is prohibited.

### 4.7.9		Saltpans (see M4 - §353.7)

If the seaward edge of the saltpan is coincident with the coastline it should be encoded as a COALNE with CATCOA = 2.

The saltpan area is encoded as a LNDRGN with CATLND = 15 on top of a LNDARE (i.e. the saltpan does not form a hole in the land area).

### 4.7.10	 	Glaciers (see M4 - §353.8)

If the seaward edge of the glacier is coincident with the coastline it should be encoded as a COALNE with CATCOA = 6.

The portion of the glacier that is on land is encoded as an ICEARE with CATICE = 5 on top of a LNDARE (i.e. the glacier does not form a hole in the land area).

ICEARE objects that are located in the sea should be encoded on top of either a LNDARE or a UNSARE if the depth of water beneath them is unknown or on top of a DEPARE if the depth is known.

### 4.7.11	 	Vegetation (see M4 - §352.4 and §354)

Geo object:		Vegetation (VEGATN)

Attributes:		CATVEG		CONVIS		ELEVAT

HEIGHT -	approximate altitude of the top of vegetation

NOBJNM		OBJNAM		VERACC		VERDAT		VERLEN		INFORM		NINFOM

Isolated trees used as landmarks are encoded as VEGATN with CATVEG = 13 to 21.

A mangrove area (see M4 - §312.4) is encoded as a VEGATN with CATVEG = 7.

The seaward edge of a mangrove area is encoded as a COALNE with CATCOA = 7 and the spatial object may have the attribute QUAPOS = 4 (approximate).

### 4.7.12	 	Lava flow (see M4 - §355)

- The lava flow is encoded as a LNDRGN with CATLND = 14.

## 4.8		Artificial features

### 4.8.1		Canals (see M4 - §361)

Geo object:		Canal (CANALS)

Attributes:		CATCAN		CONDTN	DATEND		DATSTA

HORACC -	applies to HORCLR

HORCLR		HORWID		NOBJNM		OBJNAM		STATUS		INFORM		NINFOM

Remarks:

- If the canal is navigable at compilation scale it must be encoded as DEPARE or DRGARE (see clause 5.4.8) and the canal banks must be encoded as COALNE or SLCONS. If the name of the canal is required it must be encoded as a SEAARE with CATSEA = 51. It must not also be encoded as a CANALS object.

- If the canal is not navigable at compilation scale it must be encoded as a CANALS with a LNDARE or UNSARE underneath. The name of the canal may be encoded using OBJNAM.

- Use of the object class CANBNK is prohibited.

### 4.8.2		Railways (see M4 - §362)

Geo object:		Railway (RAILWY)

Attributes:		CONDTN	HEIGHT		NOBJNM		OBJNAM		STATUS		VERACC

INFORM		NINFOM

Remarks:

- Railway stations are encoded as BUISGL with FUNCTN = 8.

### 4.8.3		Tunnels (see M4 - §363.1)

Geo object:		Tunnel (TUNNEL)

Attributes:		BURDEP		CONDTN

HORACC -	applies to HORCLR

HORCLR		NOBJNM		OBJNAM		VERACC		VERCLR		STATUS

INFORM		NINFOM

Remarks:

- If there is a waterway navigable at compilation scale, inside the tunnel, this waterway is encoded as a navigable canal with DEPARE or DRGARE. There is no LNDARE in the area covering the tunnel.

- If there is a waterway which is not navigable at compilation scale, inside the tunnel, this waterway is encoded as a CANALS. The LNDARE covers the tunnel. The attributes HORACC, HORCLR, VERACC and VERCLR are not encoded.

- If there is no waterway in the tunnel (but a railway, a road...) only the TUNNEL should be encoded (not the railway or the road), covered by a LNDARE, DEPARE or DRGARE as appropriate.

### 4.8.4		Cuttings and embankments (see M4 - §363.2 and §364.1)

Cuttings and embankments are encoded like cliffs, as SLOGRD and SLOTOP (see clause 4.7.5), with CATSLO = 1 (cutting) or 2 (embankment).

### 4.8.5		Dams (see M4 - §364.2)

Geo object:		Dam (DAMCON)

Attributes:		CATDAM		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND

DATSTA		HEIGHT		NATCON		NOBJNM		OBJNAM		VERACC		VERDAT

VERLEN		INFORM		NINFOM

Remarks:

- Dams are encoded with a LNDARE underneath.

- The geometry of the dam includes any gate. The gates may be encoded separately.

### 4.8.6		Flood barrages (see M4 - §326.7)

When a flood barrage is inside an area which is navigable at compilation scale, the fixed part of the flood barrage are encoded as DAMCON with CATDAM = 3 (covered by a LNDARE) and the opening part is encoded as a GATCON with CATGAT = 2 (covered with DEPARE).

When a flood barrage is inside an area not navigable at compilation scale, the gates need not be encoded. In this case the DAMCON goes right across the river.

### 4.8.7		Dykes (see M4 - §313.1)

Geo object:		Dyke (DYKCON)

Attributes:		CONDTN	CONRAD	DATEND		DATSTA		HEIGHT		NATCON		VERACC

VERDAT		VERLEN		INFORM		NINFOM

Remarks:

- When a dyke is coincident with the coastline, it must be encoded as a DYKCON and in addition a SLCONS of type line, without CATSLC, along its seaward border.

- When the DYKCON is of type area, it must have a LNDARE underneath.

- At large scale, the dyke crown (the topline of the dyke) may be encoded as a SLOTOP with CATSLO = 2 (embankment).

### 4.8.8		Roads and tracks (see M4 - §365)

Geo object:		Road  (ROADWY)

Attributes:		CATROD		CONDTN	NATCON		NOBJNM		OBJNAM		STATUS		INFORM

NINFOM

Remarks:

- Road crossings (CATROD = 7) should not be encoded.

### 4.8.9		Causeways

Geo object:		Causeway (CAUSWY)

Attributes:		CONDTN	NATCON		NOBJNM		OBJNAM		STATUS		WATLEV		INFORM

NINFOM

### 4.8.10	 	Bridges (see M4 - §381)

Geo object:		Bridge (BRIDGE)

Attributes:		CATBRG		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND

DATSTA

HORACC -	applies to HORCLR

HORCLR		NATCON		NOBJNM		OBJNAM

VERACC - 	applies to VERCCL, VERCLR, VERCOP

VERCCL		VERCLR		VERCOP

VERDAT - 	applies to VERCCL, VERCLR, VERCOP

INFORM		NINFOM

Remarks:

- Water under a bridge is encoded in the normal way, with DEPCNT and DEPARE, DRGARE or UNSARE if it is navigable at compilation scale, or with a LNDARE or UNSARE if it not navigable.

- In navigable water, bridge supports must be encoded where possible as PYLONS (see clause 4.8.18) with CATPYL = 4 or 5.

- It is not mandatory to encode the road on the bridge.

### 4.8.11	 	Conveyors (see M4 - §382.3)

Geo object:		Conveyor (CONVYR)

Attributes:		CATCON		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND

DATSTA		HEIGHT		LIFCAP		NOBJNM		OBJNAM		PRODCT		STATUS

VERACC - 	applies to VERCLR (not HEIGHT)

- VERCLR

VERDAT - 	applies to VERCLR (not HEIGHT)

VERLEN		INFORM		NINFOM

### 4.8.12	 	Airfields (see M4 - §366)

At large scale the airport may be encoded with the following objects: AIRARE (area), RUNWAY (area or line), BUISGL (area or point) or LNDMRK (area or point). At least one AIRARE or RUNWAY must be in this set of objects. All these objects may be associated using a collection object C_ASSO.

At small scale the airport is encoded as an AIRARE of type point.

Geo object:		Airport / airfield (AIRARE)

Attributes:		CATAIR		CONDTN	CONVIS		NOBJNM		OBJNAM		STATUS		INFORM

NINFOM

Geo object:		Runway (RUNWAY)

Attributes:		CATRUN		CONDTN	CONVIS		NATCON		NOBJNM		OBJNAM		PEREND

PERSTA		STATUS		INFORM		NINFOM

Remarks:

- If some buildings are visually conspicuous, they must be encoded as separate objects.

- Two crossing runways can be encoded as one area.

- The control tower is encoded as LNDMRK with FUNCTN = 39 and CATLMK = 17. The other buildings are encoded as BUISGL.

- Seaplane landing areas are encoded as SPLARE.

### 4.8.13		Production and storage areas (see M4 - §367)

Geo object:		Production / storage area (PRDARE)

Attributes:		CATPRA -	1 - quarry

2 - mine

3 - stockpile

4 - power station area

5 - refinery area

6 - timber yard

7 - factory area

8 - tank farm

9 - wind farm

10 – slag heap/spoil heap

CONDTN	CONRAD	CONVIS		DATEND		DATSTA		ELEVAT		HEIGHT

NOBJNM		OBJNAM		PRODCT		STATUS		VERACC		VERDAT		VERLEN

INFORM		NINFOM

Remarks:

- It there are identified buildings or equipment in this area, they are encoded as BUISGL, CRANES, LNDMRK, SILTNK...

- If visible from seaward, the quarry face is encoded like a cliff (see clause 4.7.5).

### 4.8.14		Built-up areas (see M4 - §370)

Geo object:		Built-up area (BUAARE)

Attributes:		CATBUA		CONDTN	CONRAD	CONVIS		HEIGHT		NOBJNM		OBJNAM

VERACC		VERDAT		INFORM		NINFOM

Remarks:

- A built-up area crossed by line objects (roads, streets, railways...) is not divided into multiple objects, unless separate sections of the built-up area have at least one different attribute value.

- However, for the purposes of presentation, rivers or canals of type area, must divide a built-up area into several objects.

- Use of the object class SQUARE is prohibited.

- Several buildings or built-up areas may be referred to by the same settlement, village or town name on the source. In such cases the individual buildings or built up areas should be encoded as separate unnamed BUISGL or BUAARE, and additionally an ADMARE covering the whole named area should be created with the name encoded in OBJNAM. The ADMARE should also have the attribute JRSDTN = 3.

### 4.8.15		Buildings, landmarks, tanks, silos (see M4 - §370 to §378)

Constructions may be encoded as BUISGL, LNDMRK or SILTNK, depending of their shape and use. The following table describes the way to encode them.

Fencelines are encoded as FNCLNE (see clause 4.8.16).

Geo object:		Building, single (BUISGL)

Attributes:		BUISHP		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		ELEVAT

FUNCTN	 	HEIGHT		NATCON		NOBJNM		OBJNAM		STATUS		VERACC

VERDAT		VERLEN		INFORM		NINFOM

Geo object:		Landmark (LNDMRK)

Attributes:		CATLMK		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		ELEVAT

HEIGHT		NATCON		NOBJNM		OBJNAM		STATUS		FUNCTN		VERACC

VERDAT		VERLEN		INFORM		NINFOM

Geo object:		Silo / tank (SILTNK)

Attributes:		BUISHP		CATSIL		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS

ELEVAT		HEIGHT		NATCON		NOBJNM		OBJNAM		PRODCT		STATUS

VERACC		VERDAT		VERLEN		INFORM		NINFOM

In the following table the symbol ‘/’ means that this attribute is not relevant. A blank means that the encoder can chose a value.

| Feature | INT1 | Object-class | FUNCTN | CATLMK | BUISHP | PRODCT | CATSIL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Administrative | IG72 | BUISGL | 18 | / |  | / | / |
| Bank office |  | BUISGL | 13 | / |  | / | / |
| Buddhist temple | IE16 | BUISGL | 25 | / |  | / | / |
| Bus station |  | BUISGL | 42 | / |  | / | / |
| Cairn | IQ100 | LNDMRK | / | 1 | / | / | / |
| Cemetery | IE19 | LNDMRK |  | 2 | / | / | / |
| Chapel | IE11 | BUISGL | 21 | / |  | / | / |
| Chimney | IE22 | LNDMRK |  | 3 |  | / |  |
| Church | IE10.1 | BUISGL | 20 | / |  | / | / |
| Church dome, cupola | IE10.4 | LNDMRK | 20 | 15 | / | / | / |
| Church spire | IE10.3 | LNDMRK | 20 | 20 | / | / | / |
| Church tower | IE10.2 | LNDMRK | 20 | 17 | / | / | / |
| Clock tower |  | LNDMRK | 38 | 17 | / | / | / |
| Column | IG66 | LNDMRK | / | 10 | / | / | / |
| Communication mast |  | LNDMRK | 29 | 7 | / | / | / |
| Communication tower |  | LNDMRK | 29 | 17 | / | / | / |
| Control tower |  | LNDMRK | 39 | 17 | / | / | / |
| Cooling tower |  | LNDMRK | 35 | 17 | / | / | / |
| Cross, calvary | IE12 | LNDMRK |  | 14 | / | / | / |
| Custom office | IF61 | BUISGL | 3 | / |  | / | / |
| Dish aerial | IE31 | LNDMRK |  | 4 | / | / | / |
| Dome or cupola, part of a building |  | LNDMRK |  | 15 | / | / | / |
| Educational facility |  | BUISGL | 19 | / |  | / | / |
| Factory | IG80 | BUISGL | 16 | / |  | / | / |
| Flagstaff, flagpole | IE27 | LNDMRK |  | 5 | / | / | / |
| Flare stack | IE23 | LNDMRK |  | 6 | / | / | / |

| Feature | INT1 | Object-class | FUNCTN | CATLMK | BUISHP | PRODCT | CATSIL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| grain elevator |  | SILTNK | / | / |  | 22 | 3 |
| harbour-master’s office | IF60 | BUISGL | 2 | / |  | / | / |
| Headquarters for district control |  | BUISGL | 14 | / |  | / | / |
| health office | IF62.1 | BUISGL | 4 | / |  | / | / |
| Hospital | IF62.2 | BUISGL | 5 | / |  | / | / |
| Hotel | ID6 | BUISGL | 7 | / |  | / | / |
| house, building | IG63 | BUISGL |  | / |  | / | / |
| light house (tower) | IP1 | LNDMRK | 33 | 17 | / | / | / |
| light house (other shapes) | IP1 | BUISGL | 33 | / |  | / | / |
| lookout station in general | IG77 | BUISGL | 28 | / |  | / | / |
| lookout tower |  | LNDMRK | 28 | 17 | / | / | / |
| marabout | IE18 | BUISGL | 27 | / |  | / | / |
| mast in general | IG67 | LNDMRK |  | 7 | / | / | / |
| memorial plaque |  | LNDMRK |  | 11 | / | / | / |
| microwave tower |  | LNDMRK | 34 | 17 | / | / | / |
| minaret | IE17 | LNDMRK | 26 | 20 | / | / | / |
| monument | IE24 | LNDMRK |  | 9 | / | / | / |
| mooring mast | IG69 | LNDMRK | 40 | 7 | / | / | / |
| mosque | IE17 | BUISGL | 26 | / |  | / | / |
| obelisk |  | LNDMRK | / | 12 | / | / | / |
| observation tower |  | LNDMRK | 36 | 17 | / | / | / |
| pagoda | IE14 | BUISGL | 23 | / |  | / | / |
| pilot lookout | IT2 | BUISGL | 12 | / |  | / | / |
| pilot office | IT3 | BUISGL | 11 | / |  | / | / |
| police station |  | BUISGL | 9 | / |  | / | / |
| post office | IF63 | BUISGL | 6 | / |  | / | / |
| power station | IG88 | BUISGL | 17 | / |  | / | / |
| radar dome | IE30.4 | LNDMRK | 32 | 15 | / | / | / |
| radar mast | IE30.1 | LNDMRK | 32 | 7 | / | / | / |
| radar scanner | IE30.3 | LNDMRK |  | 16 | / | / | / |
| radar tower | IE30.2 | LNDMRK | 32 | 17 | / | / | / |

| Feature | INT1 | Object-class | FUNCTN | CATLMK | BUISHP | PRODCT | CATSIL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| radio mast | IE28 | LNDMRK | 31 | 7 | / | / | / |
| radio tower | IE29 | LNDMRK | 31 | 17 | / | / | / |
| railway station | ID13 | BUISGL | 8 | / |  | / | / |
| large rock or boulder on land |  | LNDMRK | / | 21 | / | / | / |
| shinto shrine | IE15 | BUISGL | 24 | / |  | / | / |
| Silo | IE33 | SILTNK | / | / |  |  | 1 |
| spire, part of a building |  | LNDMRK |  | 20 | / | / | / |
| Stadium |  | BUISGL | 41 | / |  | / | / |
| Statue |  | LNDMRK |  | 13 | / | / | / |
| Tank | IE32 | SILTNK | / | / |  |  | 2 |
| television mast | IE28 | LNDMRK | 30 | 7 | / | / | / |
| television tower | IE29 | LNDMRK | 30 | 17 | / | / | / |
| Temple | IE13 | BUISGL | 22 | / |  | / | / |
| timeball tower |  | LNDMRK | 37 | 17 | / | / | / |
| Tower | IE20 | LNDMRK |  | 17 | / | / | / |
| tower, part of a building |  | LNDMRK |  | 17 | / | / | / |
| transit shed, warehouse | IF51 | BUISGL | 15 | / |  | / | / |
| water tower | IE21 | SILTNK | / | / | / | 3 or 8 | 4 |
| water-police station |  | BUISGL | 10 | / |  | / | / |
| Windmill | IE25 | LNDMRK |  | 18 | / | / | / |
| windmotor | IE26 | LNDMRK |  | 19 | / | / | / |
|  |  |  |  |  |  | table 4.1 |  |  |

Remarks:

- A ruined building or landmark is encoded is the same way as the feature in good condition, but with CONDTN = 2.

- When a building is shown as an area, indicating its true shape, with a prominent feature such as a tower or spire, two objects are encoded (see Figure 3 b):

-	a BUISGL of type area for the main building,

-	a LNDMRK of type point for the specific part.

Figure 3 - Landmarks

### 4.8.16		Fences	 and walls

Geo object:		Fence/wall (FNCLNE)

Attributes:		CATFNC		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		ELEVAT

HEIGHT		NATCON		NOBJNM		OBJNAM		STATUS		VERACC		VERDAT

VERLEN		INFORM		NINFOM

### 4.8.17		Fortified structures (see M4 - §379)

Geo object:		Fortified structure (FORSTC)

Attributes:		CATFOR		CONDTN	CONRAD	CONVIS		HEIGHT		NATCON		NOBJNM

OBJNAM		VERACC		VERDAT		VERLEN		INFORM		NINFOM

### 4.8.18		Pylons and bridge supports (see M4 - §381 and §382)

Geo object:		Pylon / bridge support (PYLONS)

Attributes:		CATPYL		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND

DATSTA		HEIGHT		NATCON		NOBJNM		OBJNAM		VERACC		VERDAT

VERLEN		WATLEV		INFORM		NINFOM

Remarks:

- Area PYLONS must be covered by an area LNDARE. See also clause 4.8.10.

### 4.8.19		Oil barriers

A construction to dam the flow of oil on water is encoded as:

Geo object:		Oil barrier (OILBAR)

Attributes:		CATOLB		CONDTN	DATEND		DATSTA		NOBJNM

OBJNAM		STATUS		INFORM		NINFOM

### 4.8.20		Views and sketches, viewpoints

When a drawing or a photograph is available, the attribute PICREP is used to indicate the file name (without the path) of the external graphic file. Picture files which form part of the ENC must be in TIFF format. Picture files in other formats may be used if a private agreement exists between the data producer and receiver. An ECDIS may not be able to display files in formats other than TIFF.

### 4.8.21 	Signs and Notice boards

Geo objects:		Beacon, special purpose (BCNSPP)

Buoy special purpose (BOYSPP)

Attributes:		BCNSHP – only for BCNSPP

BOYSHP – only for BOYSPP

CATSPM 	COLOUR	COLPAT		CONDTN – only for BCNSPP	CONRAD						CONVIS- only for BCNSPP	DATEND		DATSTA

ELEVAT		HEIGHT – only for BCNSPP

MARSYS -	only if different to the value encoded in M_NSYS

NATCON		NOBJNM		OBJNAM		PEREND		PERSTA

STATUS		VERACC		VERDAT		VERLEN		INFORM

NINFOM

Remarks:

- ELEVAT may encode the altitude of the ground level, but only for beacons built on land.

- HEIGHT encodes the total altitude of the beacon, including the equipment objects (topmark, light...), over the vertical datum.

- VERLEN encodes the total vertical length of the beacon, including the equipment objects (topmark, light...), above the seabed or ground. For a buoy VERLEN encodes the total vertical length, including the equipment objects (topmark, light...), above the water level.

- If a beacon or buoy has more than one colour, the attribute COLOUR should be encoded according to the rules laid out in clause 2.4.

- Any text shown on a notice board or sign should be encoded in INFORM.

- The shape and colour of a notice board may be indicated by encoding the board as a DAYMAR.

# 5.			Depth

Figure 4 – Depth contours

## 5.1		Sounding datum

- See clause 2.1.3.

## 5.2		Depth contours (see M4 - §411)

Geo object:	Depth contour (DEPCNT)

Attributes:	VALDCO -	value of depth contour, negative value for drying contours.

VERDAT	INFORM	NINFOM

The boundary of a drying rocky area (see INT1 - IJ20) or coral reef (see INT1 - IJ22) may be coincident with the zero meter contour (see ‘fg’ in Figure 4). This boundary is encoded with a DEPCNT, VALDCO = 0.

On paper charts, the presentation of contours in areas of steep slope is sometimes generalised so that closely spaced contours are removed to leave only one (see ‘ab’ in Figure 4). In such cases the contour must be encoded with the shallowest depth. A linear depth area must also be created at the same position (see clause 5.4.3).                           Wherever possible, contours must be closed or connected to the border of the cell or another contour in order to define closed areas.

Spatial objects associated with approximate contours (see M4 - §411.2) are encoded with QUAPOS = 4.

## 5.3		Soundings (see M4 - §412)

Geo object:		Sounding (SOUNDG)

Attributes:		EXPSOU -	1 - the usual case

2 - for some soundings or dangers reported, but not confirmed, whose value

is shallower than the normal range of depth area surrounding it.

3 - for some rare soundings showing deeps of limited area, where contours

are missing.

NOBJNM		OBJNAM

QUASOU -	see table below.

SOUACC -	see use of M_QUAL (clause 2.2.3.1 and 2.2.3.4)

STATUS - 	18 - for existence doubtful

TECSOU -	only for lower reliability soundings

VERDAT		INFORM		NINFOM

SORDAT - 	see table below.

| Sounding | M4 | INT 1 | QUAPOS | QUASOU | Remarks |
| --- | --- | --- | --- | --- | --- |
| in true position | 412.1 | II10 |  | 1 | Generally QUAPOS = 10 |
| out of position on paper chart | 412.2 | II11 II12 |  | 1 | Spatial object is encoded at the true position. There is no sounding, out of position in an ENC. |
| no bottom found | 412.3 | II13 |  | 5 |  |
| lower reliability | 412.4 | II14 | 4 | 4 |  |
| Drying | 413 | II15 |  | 1 | Negative value |
| Doubtful | 424.4 | II2 |  | 3 | Existence doubtful is encoded using STATUS = 18 |
| reported but not confirmed |  | II4 | 8 | 9 | The year of report is encoded in SORDAT |
|  |  |  |  |  | table 5.1 |

A sounding associated with rock or coral which is an obstruction to navigation must be encoded as UWTROC (INT1 IK14) with VALSOU populated with the value of the sounding.

Geometry of soundings is carried by a 3 dimensional array (latitude, longitude, depth). In the interests of efficiency, multiple soundings must be encoded in one spatial object, provided that all the spatial and geo object attributes are common to the group.

As the sounding multiplication factor (SOMF)  for  ENC is always 10,  soundings may only be given to one decimal place. Drying soundings are indicated by a negative value.

- For soundings surrounded by a danger line, see clause 6.3.

## 5.4		Depth areas

### 5.4.1		Geo object depth area

The sea area, the intertidal area and the navigable parts of rivers, lakes and canals must be divided into depth areas, each of them having a range of depth.

As many depth areas as possible must be created using charted depth contours.

In the maritime area, if there is a discontinuity in the succession of minimum and maximum depths of adjoining Group 1 area objects (e.g. DRVAL1 and DRVAL2 for DEPARE and DRGARE or height of the coastline (-H) for other Group 1 objects), a DEPARE of type line must be created to fill this gap. A line DEPARE encodes the case where two or more of the contours normally shown on the chart merge to form a single cliff contour (at the scale of the chart). This may happen naturally at sea, or at a dredged area boundary, at a coastline or shoreline construction, etc. However, DEPARE of type line may also be created at borders of area object of class DEPARE and DRGARE when there is no discontinuity in the succession of their minimum and maximum depth and the limit between the two objects does not contain any line object of class COALNE, DAMCON, DEPCNT, GATCON, LNDARE or SLCONS.

DEPARE of type line may also be created at the borders of area objects FLODOC, HULKES, PONTON, UNSARE and LNDARE (in all such cases the DRVAL1 of the DEPARE will be -H).

- DEPARE of type area are part of Group 1.

Geo object:		Depth area (DEPARE)

Attributes:		DRVAL1 -	 (see 5.4.3)

DRVAL2 - 	(see 5.4.3)

QUASOU	SOUACC	VERDAT		INFORM		NINFOM

### 5.4.2		Geometry of depth areas

Where areas are not closed on paper charts, it may be necessary to close these areas using edges without associated line objects. This is mandatory at the boundary of a cell (see Figure 5).

Remarks:

Figure 5 – Geometry of depth areas

- For short isolated sections of DEPCNT such as (bc), it is up to the encoder or Hydrographic Office whether to encode such small DEPARE as areas, or only the DEPCNT as floating lines.

### 5.4.3		Use of DRVAL1 and DRVAL2 for depth areas in general

For each depth area of type area, DRVAL1 and DRVAL2 are encoded with the values corresponding to the shallowest and deepest depth of that area. These values must be chosen from the values of the standard depth contours normally shown on the chart .

The overall succession of DRVAL1 and DRVAL2 in the whole maritime area must be continuous (no gaps,  no overlaps). Therefore, for each DEPARE of type line, DRVAL1 usually takes the maximum value  (DRVAL2) of the shallowest adjoining Group 1 area (DEPARE or DRGARE), and DRVAL2 takes the minimum value (DRVAL1) of the deepest adjoining DRGARE or DEPARE of type area. However, in some cases DRVAL1 and DRVAL2 may be set to the same value, dependant upon the compilation of the source data. But if this shallowest adjoining Group 1 area is FLODOC, HULKES, LNDARE, PONTON or UNSARE, the value of DRVAL1 of the line DEPARE is equal to -H.

Drying areas within which a drying height is indicated without a true position should be encoded as DEPARE with DRVAL1 and DRVAL2 set to standard contour values or -H. The drying height should be encoded in INFORM.

Water areas shown behind shoreline constructions to which there is no access and where the true depth is unknown (and which do not qualify as either lakes, docks, rivers, canals or lock basins), should be encoded as UNSARE. If the water depth is known these areas should be encoded as DEPARE in the normal way.

Figure 6 – Depth areas

(a): H = Height of the coastline datum above sounding datum or rounded value (e.g.: (1) the value of the highest drying contour indicated on the source document; (2) zero if the coastline datum is also the sounding datum).

- In the following clauses, the paragraph numbers refer to the item numbers in Figure 6.

- 1.	If the area is limited by two or more depth contours:

DRVAL1 takes the value of the depth contour immediately shallower than the value of DRVAL2.

DRVAL2 take the value of the deepest depth contour bounding the area.

- If the deepest depth is shown by a depth contour, and the shallowest depth is shown by a sounding  (a shoal):

DRVAL1 takes the value of the standard depth contour immediately shallower than the value of the sounding or - height of the coastline datum above sounding datum.

DRVAL2 takes the value of the depth contour.

3.	If the deepest depth is shown by a sounding and the shallowest depth is shown by a depth contour (a deep):

DRVAL1 takes the value of the depth contour

DRVAL2 takes the value of the standard depth contour immediately deeper than or equal to the value of the sounding.

- 4.	If the shallowest depth is defined by the coastline:

DRVAL1 = - height of the coastline datum above sounding datum

DRVAL2 takes the value of the shallowest standard depth contour.

- 5.	If the area is limited by only one depth contour and contains no soundings and it is a shoal:

DRVAL1 takes the value of the standard depth contour immediately shallower than the value of the depth contour shown.

DRVAL2 takes the value of the depth contour shown.

- 6.	If the area is limited by only one depth contour and contains no soundings and it is a deep:

DRVAL1 takes the value of the depth contour shown.

DRVAL2 takes the value of the standard depth contour immediately deeper than the value of the depth contour shown.

- 7.	If two or more depth contours are merged, a line DEPARE is created at the same position:

DRVAL1 takes the value of the shallowest of the merged depth contours.

DRVAL2 takes the value of the deepest of the merged depth contours.

- 8.	If one or more depth contours are merged with the coastline, a line DEPARE is created at the same position:

DRVAL1 = - height of the coastline datum above sounding datum.

DRVAL2 takes the value of the deepest of the merged depth contours.

- 9.	If the area is limited by an incomplete depth contour on one side (such as in an incompletely surveyed area), and a  complete depth contour on the other:

DRVAL1 takes the value of the shallowest depth contour.

DRVAL2 takes the value of the deepest depth contour.

Note: The encoding of this DEPARE as an area is optional - see figure  5.

- 10.	If the area is limited by standard depth contours but contains an incomplete depth contour:

DRVAL1 takes the value of the shallowest standard depth contour.

DRVAL2 takes the value of the deepest depth contour.

### 5.4.4		Use of DRVAL1 and DRVAL2 for depth areas adjoining non-navigable waterways.

Figure 7 – Depth areas at seaward end of a non-navigable waterway and approaches

Remarks:

- The linear DEPARE (ab), (bd), (de) may be encoded as one linear feature object (abde).

- In the following clauses, the paragraph numbers refer to the item numbers in Figure 7.

1.	If an area is intertidal and adjacent to a non-navigable waterway (RIVERS, CANALS DOCARE):

DRVAL1 = - height of the coastline datum above sounding datum

DRVAL2 takes the value of the deepest depth contour bounding the area.

2.	If an area is always under water and adjacent to a non-navigable waterway (RIVERS, CANALS, DOCARE):

- DRVAL1 = 0

- DRVAL2 takes the value of the deepest depth contour bounding the area.

3.	If an area is always under water and adjacent to a non-navigable waterway, a line DEPARE is created to close the non-navigable waterway (this border of the waterway may also be encoded as a masked line):

DRVAL1 = - height of the coastline datum above sounding datum

DRVAL2 = 0

4.	If one or more depth contours merge with the coastline, a line DEPARE is created at the same position:

DRVAL1= - height of the coastline datum above sounding datum

DRVAL2 takes the value of the deepest of the merged depth contours.

5.	Non-navigable waterways are encoded as CANALS, DOCARE or RIVERS and must  also be encoded with LNDARE or UNSARE (as an area).

### 5.4.5		Use of DRVAL1 and DRVAL2 for depth areas adjoining constructions which are always dry

### Figure 8 – Depth areas adjoining a shoreline construction

### 5.4.6		Use of DRVAL1 and DRVAL2 for depth areas adjoining a dredged area

Borders of a dredged area may be covered with line DEPARE between each intersection with a depth contour:

DRVAL1 takes the value of maximum depth of the shallowest adjoining Group 1 area object (DRVAL2 or -H)

DRVAL2 takes the value of dredging.

Figure 9 – Depth areas adjoining a dredged area

### 5.4.7		Use of DRVAL1 and DRVAL2 for dredged areas adjoining dredged areas

Figure 10 – Dredged areas adjoining dredged areas

When a dredged area is deeper than the adjoining Group 1 area object , the boundary of the dredged area is encoded as a DEPARE of type line.

DRVAL1 takes the value of maximum depth (DRVAL2 or -H) of the adjoining Group 1 area object.

DRVAL2 takes the value of the depth of dredging.

In figure 10:

(ab)	DRVAL1 =	10	DRVAL2 =	12

(cd)	DRVAL1 =	5	DRVAL2 =	8.6

(de)	DRVAL1 =	5	DRVAL2 =	9

(fg)	DRVAL1 =	5	DRVAL2 =	8.6

(hi)	DRVAL1 =	10	DRVAL2 =	12

When the depth of dredging is within the range of depths of the adjoining depth area, no geo feature object of type line is encoded on the boundary [(bc), (ei), (fh) in figure 10].

When two dredged areas adjoin:

If the depths of dredging of the two adjoining dredged areas are either side of one or more standard depth contours, the boundary is encoded as a DEPARE of type line.

DRVAL1 takes the value of the depth of dredging of the shoalest dredged area.

DRVAL2 takes the value of the depth of dredging of the deepest dredged area.

In Figure10:

(bj)	DRVAL1 =	8.6	DRVAL2 =	12

(ij)	DRVAL1 =	9	DRVAL2 =	12

(hk)	DRVAL1 =	8.6	DRVAL2 =	12

Figure 11 – Linear depth areas along dredged area boundaries

If the depths of two adjoining dredged areas are within the same range of standard depth contours, no geo feature object of type line is encoded on the boundary [(dj) in figure 10].

In figure 11, the 5 metre dredged area extends into the 5-10 metre depth area. In order to ensure that the safety contour can be drawn correctly, it is necessary to create linear depth areas [(bc) and (ah)] with both DRVAL1 and DRVAL2 set to 5 metres along the boundary of the dredged area. There are no linear depth areas between (ge) and (cd).

### 5.4.8		Rivers, canals, lakes, basins

Where these areas are navigable at compilation scale, they are defined with objects of Group 1 (DEPARE,  DRGARE or UNSARE), and coastline-type objects (COALNE or SLCONS). Area object SEAARE is used to express the nature and the name of the areas.

Where they are not navigable at compilation scale, they are described with the objects RIVERS, CANALS, LAKARE, DOCARE or LOKBSN. These objects must have a LNDARE or UNSARE underneath. Use of the object classes CANBNK, LAKSHR and RIVBNK is prohibited.

## 5.5		Dredged areas (see M4 - §414)

Geo object:		Dredged area (DRGARE)

Attributes:		DRVAL1 -	depth of dredging

DRVAL2 -	depth of dredging

NOBJNM		OBJNAM

QUASOU -	10 - depth maintained

11 - not regularly maintained

- RESTRN

SOUACC -	see use of M_QUAL

TECSOU		VERDAT

INFORM - 	to encode the date of dredging, e.g.: Dredged in 1995

NINFOM - 	to encode the date of dredging in national language, e.g.: Dragué en 1995

Remarks:

- Area DRGARE are in Group 1.

- For the encoding of  borders of a dredged area, see clauses 5.4.6 and 5.4.7.

## 5.6		Swept areas (see M4 - §415)

Geo object:		Swept area (SWPARE)

Attributes:		DRVAL1 -	swept depth

QUASOU 	SOUACC

TECSOU -	6	- swept by wire-drag

8	- swept by vertical acoustic system

13	- swept by side scan sonar

- VERDAT

INFORM -	latest date of sweeping

NINFOM

Attributes QUASOU, SOUACC and TECSOU apply to the swept area itself. The quality for spot soundings and depth contours is described by the meta object M_QUAL (see clause 2.2.3.1).Spot soundings and depth contours shown in these areas are encoded in the normal way by SOUNDG and DEPCNT.

Even if there are no spot soundings or depth contours, SWPARE must be encoded on top of DEPARE. If there is insufficient depth information to allow the DRVAL1 and DRVAL2 to be encoded for DEPARE, DRVAL1 should be set to the swept depth and DRVAL2 should be set to ‘unknown’.

Remarks:

- When a swept area occupies the entire M_QUAL and a SWPARE is not defined separately, DRVAL1 must be used to specify the swept depth. SOUACC may be used to specify the accuracy of the swept depth defined by DRVAL1. POSACC must not be used. There is no depth or positional accuracy information provided for any underlying soundings.

- When a swept area occupies the entire M_QUAL and a SWPARE is defined separately, DRVAL1 must be the same as DRVAL1 defined within SWPARE. SOUACC may be used to specify the accuracy of the swept depth. POSACC must not be used. There is no depth or positional accuracy information provided for any underlying soundings.

- When a SWPARE feature falls within the M_QUAL, SOUACC may only be used if the same depth accuracy applies to the swept depth and to the soundings outside the SWPARE. POSACC only refers to the accuracy of depths falling outside the boundaries of the swept area. There is no depth or positional accuracy information provided for any underlying soundings within the swept area.

## 5.7		Areas of continual change (see M4 - §416)

An area of continually changing depth is described by the object CTNARE. Caution notes are encoded in INFORM or TXTDSC. (See Clause 6.6)

Such an area always sits on top of DEPARE.

An area with the indication "Less water" is described by this method.

Sand waves are encoded as SNDWAV (see clause 7.2.1).

## 5.8		Areas with inadequate depth information (see M4 - §417)

### 5.8.1		Unsurveyed areas

- Areas with no bathymetric survey information are encoded as UNSARE.

Geo object:		Unsurveyed area (UNSARE)

Attributes:		INFORM		NINFOM

Remarks:

- Area UNSARE are in Group 1.

### 5.8.2		Incompletely surveyed areas

An incompletely surveyed area should be encoded as either an UNSARE, within which there may be soundings and contours (but not depth areas) or as  DEPARE objects. The DRVAL1 and DRVAL2 attributes for such depth areas should not be NULL.

The area must be covered by M_QUAL objects with suitably defined CATZOC values and further information may be given by  M_SREL objects where appropriate.

- A cautionary note should also be encoded using an area CTNARE (see clause 6.6).

### 5.8.3 		Bathymetry in areas of minimal depiction of detail

Where areas of little or no depth information exist within a specified ENC usage they should be encoded using one of the following options:

### 5.8.3.1  	Areas of omitted bathymetry

Where larger scale coverage is available, the larger scale charts should be examined to determine the shoalest DEPARE within the whole of the area. One DEPARE should then be created, with DRVAL1 and DRVAL2 attributed from the values obtained from the larger scale. Linear DEPAREs may need to be created to join the blank area with adjoining known DEPAREs.

Where larger scale coverage does not exist, a single DEPARE should be created to cover the area. The DRVAL1 of the area should be set to the shoalest value appropriate with the colour tint that is applied to it (e.g. if blue tint is used for 5-20m areas the DRVAL1 for the blank area should be set to 5m). The DRVAL2 should be set to the shoalest value of the surrounding Group 1 polygons. Linear DEPAREs may need to be created to join the area with adjoining known DEPAREs.

In either case the areas should be covered by a CTNARE, the boundary of which follows exactly the GROUP 1 objects (DEPARE, DRGARE, FLODOC, HULKES, LNDARE, PONTON or UNSARE) that surround it (see clause 2.8.2).

### 5.8.3.2  	Areas of very simplified bathymetry

In these areas, information relating to bathymetry (depth contours, dangers, rocky areas, isolated rocks, nature of the seabed, dredged areas, unsurveyed areas...) may be individually encoded.

One  object CTNARE should be created on top of the area DEPARE(s) in order to encode a cautionary note (see clause 2.8.2).

# 6.			Dangers

## 6.1		Rocks and coral reefs (see M4 - §421)

### 6.1.1		Rocks which do not cover (islets)

Area features are encoded by:

- an area object LNDARE (mandatory)

- line objects COALNE or SLCONS (mandatory)

- point objects LNDELV (optional)

Line features are encoded by:

- a line object LNDARE (mandatory)

- point objects LNDELV (optional)

Point features are encoded by:

- a point object LNDARE (mandatory)

- a point object LNDELV (optional)

### 6.1.2		Rocks which may cover

- These rocks may cover and uncover, may be awash, or may be always underwater.

Geo object:		Underwater / awash rock (UWTROC)

Attributes:		EXPSOU -	1 - in general

2 - for rocks whose least depth may be shallower than the normal range of 							     depth area surrounding it.

- NATQUA

NATSUR -	9 - rock

14 - coral

NOBJNM		OBJNAM

QUASOU -	see table below.

SOUACC -	see use of M_QUAL

STATUS  -  18 - existence doubtful

TECSOU

VALSOU -	see table below.

- VERDAT

WATLEV -	see table below.

INFORM		NINFOM

SORDAT -	year of report, for reported but not confirmed danger

| Rock or coral reef | INT 1 | WATLEV | QUASOU | Comment |
| --- | --- | --- | --- | --- |
| Which cover and uncover, depth unknown | IK11 | 4 | 2 |  |
| Which cover and uncover, depth known | IK11 | 4 | any value but 2 | negative value for VALSOU |
| Awash | IK12 | 5 |  |  |
| Underwater rock, depth unknown | IK13 | 3 | 2 |  |
| Underwater rock, depth known | IK14 | 3 | any value but 2 |  |
|  |  |  |  | table 6.1 |

Remarks:

- A rock represented by a spot sounding and an associated nature of seabed (underwater rock not dangerous to surface navigation) may be encoded by a single object UWTROC.

- Area features that cover and uncover are encoded as DEPARE.

- When a group of rocks is surrounded by a danger line, each rock is encoded as a separate UWTROC inside an obstruction area (OBSTRN).

## 6.2		Wrecks, foul areas and obstructions (see M4 - §422)

### 6.2.1		Wrecks

Geo object:		Wreck (WRECKS)

Attributes:		CATWRK -	see table below

CONRAD	CONVIS

EXPSOU -	generally used if VALSOU is known

1 - if DRVAL1<VALSOU  DRVAL2

2 - if DRVAL1  VALSOU

3 - if VALSOU > DRVAL2

HEIGHT -	if WATLEV = 2

NOBJNM		OBJNAM

QUASOU -	see table below

SOUACC -	see use of M_QUAL

STATUS -	13 - for historic wrecks

18 - for existence doubtful

TECSOU -	see table below

VALSOU		VERACC		VERDAT		VERLEN

WATLEV - 	see table below

INFORM		NINFOM

| Wrecks... | M4 | INT 1 | CATWRK | WATLEV | QUASOU | TECSOU |
| --- | --- | --- | --- | --- | --- | --- |
| showing any part of hull or superstructure | 422.2 | IK24 IK20 | 5 | 2 or 4 | / | / |
| covers and uncovers | 422.2 | IK24 IK21 | 4 or 5 | 4 |  |  |
| Awash |  |  |  | 5 |  |  |
| the mast only is visible at high water | 422.5 | IK25 | 4 or 5 | 2 | / | / |
| the mast only is visible at low water | 422.5 | IK25 | 4 | 4 |  |  |
| measured depth | 422.4 | IK26 | 1 or 2 | 3 | 6 |  |
| depth measured and swept by wire drag | 422.3 | IK27 | 1 or 2 | 3 | 6 | 6 |
| depth unknown, considered dangerous by the responsible hydrographic office. | 422.5 | IK28 | 2 | 3 | 2 | / |
| depth unknown, not considered dangerous by the responsible hydrographic office. | 422.6 | IK29 | 1 | 3 | 2 | / |
| depth unknown, with a safe clearance | 422.7 | IK30 | 1 or 2 | 3 | 7 | / |
| remains of wrecks | 422.8 | IK31 | 3 |  |  |  |
|  |  |  |  |  |  | table 6.2 |

Where a wreck is shown with its true shape (large scale chart):

Soundings and heights are often given inside a wreck to show the highest points of the hull or superstructures (mast, funnel...).

These features are encoded by:

- An area object WRECKS with all attributes applying to the highest point of the wreck.

- Point objects LNDELV to encode the features of wrecks that are always dry ; the type of each feature (mast, funnel...) may be encoded in INFORM and NINFOM.

- Point objects SOUNDG to encode the features of wrecks which cover, or cover and uncover ; the type of each feature (mast, funnel...) may be encoded in INFORM and NINFOM, which means that these soundings are encoded individually.

Remarks:

- Area WRECKS must be covered by an area object from Group 1 (DEPARE or UNSARE as appropriate)

- For a wreck whose true depth is unknown but for which there is a safe clearance, the safe clearance depth should be encoded in the attribute VALSOU.

### 6.2.2		Obstructions and foul areas

Snags, stumps, wellheads, diffusers, cribs, fish havens, foul areas, foul grounds, booms, ice booms and ground tackle are encoded as obstructions.

Geo object:		Obstruction (OBSTRN)

Attributes:		CATOBS	CONDTN

EXPSOU -	generally used if VALSOU is known:

1 - if DRVAL1<VALSOU  DRVAL2

2 - if DRVAL1  VALSOU

3 - if VALSOU > DRVAL2

HEIGHT -	only if WATLEV = 1 or 2

NATCON		NATQUA		NATSUR		NOBJNM		OBJNAM

PRODCT -	only used for wellheads

QUASOU -	see table below

SOUACC -	see use of M_QUAL

STATUS -	18 - existence doubtful

TECSOU -	see table below

VALSOU		VERACC		VERDAT

VERLEN -	distance above the sea bed

WATLEV -	see table below

INFORM		NINFOM

| Obstruction... | INT 1 | WATLEV | QUASOU | TECSOU |
| --- | --- | --- | --- | --- |
| depth unknown | IK40 | 3 or 4 | 2 | / |
| least depth known | IK41 | 3 or 4 | 6 |  |
| swept by wire to the depth shown | IK42 | 3 | 6 | 6 |
|  |  |  |  | table 6.3 |

Remarks:

- If the nature of a dangerous underwater object, dangerous underwater area, or floating object is not explicitly known, it must be encoded as an OBSTRN.

- Area OBSTRN must be covered by an area object from Group 1 (DEPARE or UNSARE as appropriate).

## 6.3		Danger lines

### 6.3.1		Danger line around a point danger or an isolated sounding.

In general terms a danger line that surrounds a single symbol or sounding (e.g. INT1 - IK28, IK40a or IK 41 to IK43.1) should not be encoded as a separate area. However, when the danger line indicates the true shape of the feature it should be encoded as an area WRECKS or OBSTRN. A single sounding enclosed by a danger line should be encoded as a point OBSTRN. The sounding value is encoded in the attribute VALSOU.

### 6.3.2		Danger line limiting an area of wrecks or obstructions

Figure 12 – Area of wrecks

The area is encoded as an area object WRECKS or OBSTRN, with the attribute values from the shallowest object in this area. The area must also be covered by a DEPARE or UNSARE object as appropriate.

To code one or more least depth in this area, each of them is described by a point object, in addition to the area object.

### 6.3.3		Danger line limiting an area through which navigation is not safe (see M4 -§420.1)

A danger line on a paper chart, limiting an area through which navigation is not safe, is encoded as an area obstruction OBSTRN with CATOBS = 6.

## 6.4		Overfalls, races, breakers, eddies (see M4 - §423)

Geo object:		Water turbulence (WATTUR)

Attributes:		CATWAT		OBJNAM		NOBJNM		INFORM		NINFOM

Remarks:

- A breaker over an off-lying shoal, represented on the paper chart with the abbreviation "Br" near the danger, is encoded by a WATTUR with the same position as the feature causing the breaker.

- If an area covered by breakers is considered to be unsurveyed it should be encoded as a UNSARE.

## 6.5		Doubtful dangers (see M4 - §424)

The fact that a danger is doubtful is indicated with the feature attributes QUASOU and STATUS and the spatial attribute QUAPOS.

|  | M4 | INT 1 | QUAPOS | QUASOU | STATUS |
| --- | --- | --- | --- | --- | --- |
| Position approximate | 424.1 | IB7 | 4 |  |  |
| Position doubtful | 424.2 | IB8 | 5 |  |  |
| Existence doubtful | 424.3 | II1 |  |  | 18 |
| Doubtful sounding | 424.4 | II2 |  | 3 |  |
| Reported danger | 424.5 | II3 |  | 9 |  |
|  |  |  |  |  | table 6.4 |

Remarks:

- The same notions of approximate or doubtful positions and doubtful existence also apply to features other than dangers (e.g. landmarks, buoys ...).

- The text "Discoloured water" indicates the probable existence of shoal water. This is encoded as a CTNARE with INFORM (or TXTDSC) containing a cautionary note (see clause 6.6).

## 6.6		Caution area

An area in which the mariner has to be aware of cicumstances influencing the safety of navigation (e.g. an area of continually changing depths ...) Is encoded using the object class CTNARE. This object class may be required to identify a danger, a risk, a rule or advice which is not directly related to a particular object.

Geo Object:		Caution area (CTNARE)

Attributes:		DATEND		DATSTA		PEREND		PERSTA		INFORM		NINFOM

# 7.			Nature of the seabed

## 7.1 		Description of the bottom (see M4 - §425)

Geo object:		Seabed area (SBDARE)

Figure13 - Seabed areas

Attributes:		COLOUR	NATQUA		NATSUR

WATLEV		NOBJNM		OBJNAM

INFORM		NINFOM

Remarks:

a.	Mixed natures: The dominant nature shall be given first. When there are qualifying terms associated with the various nature of seabed, the qualifying terms must be listed in the same order as the nature of surface list. Where a particular nature of surface has no qualifying term the place in the list must be left empty and the delimiting comma must be encoded. For example, to encode a bottom quality such as "fine sand, mud and broken shells": NATSUR = 4,1,17 NATQUA=1,,4

b.	Underlying material: They are encoded in the same way as mixed nature, replacing the comma by a slash (/). The surface layer is given first, followed by underlying layers.

c.	Coral reef which is always covered, represented as an area (IK16): In this area some point dangers may be shown. An area OBSTRN shall be created with CATOBS = 6 and NATSUR = 14. A point object UWTROC is created for each individual point danger.

d.	Hard bottom: The attribute NATQUA = 10 is used without being associated with the attribute NATSUR.

e.	On paper charts, in the intertidal area or along the drying line the nature of surface is sometimes shown by an open line rather than a closed area. In such cases the object SBDARE may be encoded as a linear object.

- A rock which is dangerous to navigation is encoded as UWTROC, while a rocky nature of seabed is encoded as a SBDARE.

- An object SBDARE of type area in intertidal areas, should be created  with WATLEV = 4 (covers and uncovers).

## 7.2		Special bottom types

### 7.2.1		Sand waves (see M4 - §428.1)

Geo object:		Sand waves (SNDWAV)

Attributes:		VERACC

VERLEN -	amplitude above the bottom

INFORM		NINFOM

### 7.2.2		Weed - Kelp (see M4 - §428.2)

Geo object:		Weed / kelp (WEDKLP)

Attributes:		CATWED	NOBJNM		OBJNAM		INFORM		NINFOM

### 7.2.3		Springs in the seabed (see M4 - §428.3)

Geo object:		Spring (SPRING)

Attributes:		NOBJNM		OBJNAM		INFORM		NINFOM

### 7.2.4		Tideway

A natural water course in an intertidal area where water flows during the ebb or flow of the tide, or a channel through which a tidal current runs, is encoded as:

Geo object:		Tideway (TIDEWY)

Attributes:		NOBJNM		OBJNAM		INFORM		NINFOM

Remarks:

- This object must be on top of objects of Group 1 (DEPARE ...)

8.			Sea areas

Undersea features and sea areas in general, including intertidal areas, may be described by their names (see M4 - §550) and may be limited by the spatial objects used by other geo objects (e.g. depth contours, coastlines...).

Geo object:		Sea area (SEAARE)

Attributes:		CATSEA		NOBJNM		OBJNAM		INFORM		NINFOM

Remarks:

- The area should be limited, if possible, by existing lines used by other objects (DEPCNT, COALNE...). If necessary they may be limited by other lines created to close the area or to describe a new one.

- For seas, oceans, gulfs... there is no specific value of CATSEA, therefore the generic term 'sea', 'ocean', 'gulf'... may be included in OBJNAM and NOBJNM.

- Sea areas may overlap.

- This object class has a use similar to LNDRGN (see clause 4.7.1) but for the sea.

# 9.			Harbour regulations

## 9.1		Regulations within harbour limits

### 9.1.1		Administrative harbour areas (see M4 - §430.1)

Geo object:		Harbour area (HRBARE)

Attributes:		NOBJNM		OBJNAM		STATUS		INFORM		NINFOM

Remarks:

- A masked line may be used to suppress the symbolisation of the boundary where such symbolisation is considered inappropriate.

### 9.1.2		Speed limits (see M4 - §430.2)

Speed is often limited inside harbours to prevent wakes. This is encoded as RESARE with CATREA = 24 (no wake area)  and RESTRN = 13 (no wake). In cases where speed limit is known, the attribute RESTRN = 27 (speed restricted) is used and the speed limit encoded within INFORM e.g. "Speed limit is 5 knots").

Generally the object marking the RESARE with speed limits is encoded using BCNSPP or BOYSPP with the attribute CATSPM = 25 (speed limit mark).

## 9.2		Anchorages and prohibited anchorages; moorings

### 9.2.1		Anchorages (see M4 - §431.1 and 431.3)

Geo object:		Anchorage area (ACHARE)

Attributes:		CATACH		DATEND		DATSTA		NOBJNM

OBJNAM -	name or number of the anchorage

PEREND		PERSTA		RESTRN		STATUS

INFORM -	additional information about the category of anchorage

NINFOM

Remarks:

- For individual recommended anchorages without defined limits, the associated spatial object is a point, with CATACH = 1 and STATUS = 3.

- Areas with numerous small craft moorings (see M4 - §431.7) may be encoded as ACHARE with CATACH = 8 and a spatial object of type area.

- Areas with limited time periods should be encoded with CATACH = 10 and the time period specified using inform.

- For encoding mooring buoys, see clause 9.2.4.

- Where an anchorage may only be used for a limited period (less than 24 hours) the duration should be indicated in INFORM.

### 9.2.2		Anchor berths (see M4 - §431.2)

Geo object:		Anchorage berth (ACHBRT)

Attributes:		CATACH		DATEND		DATSTA		NOBJNM

OBJNAM -	name or number of the berth

PEREND		PERSTA

RADIUS -	radius of the swinging circle in metres

STATUS

INFORM -	additional information about the category of anchorage

NINFOM

Remarks:

- If the anchor berth is defined by the centre point and a swinging circle, the associated spatial object is a point.

### 9.2.3		Anchoring restricted (see M4 - §431.4)

Areas in which anchoring is prohibited or restricted are encoded  as RESARE  (see clause 11.1)  with RESTRN = 1 (anchoring prohibited) or 2 (anchoring restricted). Additional information about the restriction is encoded in INFORM (or TXTDSC).

### 9.2.4		Mooring buoys (see M4 - §431.5)

- Mooring buoys are encoded as MORFAC (see clause 4.6.7.1) with CATMOR = 7.

9.2.5		Mooring trots (see M4 - §431.6)

Figure 14 – Mooring trot

A complete mooring trot is composed of ground tackle, mooring cables, buoys and mooring berths on junction cables.

a)	Ground tackle is encoded as OBSTRN (see clause 6.2.2) with CATOBS = 9.

b)	Mooring cables are encoded as CBLSUB (see clause 11.5.1) with CATCBL = 6.

c)	Buoys are encoded as MORFAC (see clause 4.6.7.1) with CATMOR = 7.

d)	Mooring berths are encoded as BERTHS (see clause 4.6.2).

e)	Junction cables are encoded as MORFAC (see clause 4.6.7.1) with CATMOR = 6.

- All these objects may be associated using a collection object C_AGGR.

### 9.2.6 		Anchorage - Relationship

To encode an anchorage, objects suchs as ACHARE, ACHBRT, MORFAC, RESARE, C_AGGR (mooring trots) and navigational aids may be associated using a collection object C_ASSO.

# 10.		Recommended tracks and routes

## 10.1		Leading, clearing and transit lines and recommended tracks (see M4 - §433)

Leading, clearing and transit lines may be encoded by means of NAVLNE, RECTRC and point navigational aids. This applies in the same way for visual and radio navigational aids.

Relationships are defined between these objects.

- NB. In North America the word ‘range’ is used instead of ‘transit’ and ‘leading line’.

### 10.1.1			Navigation lines and recommended tracks

Geo object:		Navigation line (NAVLNE)

Attributes:		CATNAV		DATEND		DATSTA

ORIENT -	value of the bearing from seaward

PEREND		PERSTA		STATUS

INFORM -	legend as usually shown on paper chart

NINFOM

Geo object:		Recommended track (RECTRC)

Attributes:		CATTRK		DATEND		DATSTA

DRVAL1 -	minimum depth

DRVAL2		NOBJNM		OBJNAM		ORIENT		PEREND		PERSTA		QUASOU

SOUACC	STATUS		TECSOU		TRAFIC		VERDAT

INFORM -	maximum authorised draft

NINFOM

Remarks:

- The value 3 (recommended) is not used for STATUS, in this case, because by definition a RECTRC is recommended.

- In the case of a two-way recommended track, only one value of orientation is encoded in ORIENT. It is the value of the bearing from seaward if possible. If it is not possible to define a seaward direction the value less than 180 is used.

- When the traffic is one way, the direction of digitising must reflect the direction of traffic flow.

The use of these objects is defined in the following table:

| Figure 15 |  | NAVLNE | RECTRC | Navigation aids |
| --- | --- | --- | --- | --- |
| 1 | recommended track on a leading line | CATNAV = 3 | CATTRK = 1 | at least 2 |
| 2 | clearing line on marks in line | CATNAV = 1 | none | at least 2 |
| 3 | transit line on marks in line | CATNAV = 2 | none | at least 2 |
| 4 | recommended track on a bearing | CATNAV = 3 | CATTRK = 1 | 1 |
| 5 | clearing line on a bearing | CATNAV = 1 | none | 1 |
| 6 | transit line on a bearing | CATNAV = 2 | none | 1 |
| 7 | recommended track not based on fixed marks | none | CATTRK = 2 | none |
|  |  |  |  | table 10.1 |

Figure 15 – Navigation lines and recommended tracks

Remarks:

- Even if, on the paper chart, the navigational aids are merged into only one symbol, one geo object must be created for each of them

### 10.1.2			Range systems - Relationship

To encode a range system the objects NAVLNE, RECTRC and the navigational aids must be aggregated using a collection object C_AGGR.

This aggregation object may be associated using a C_ASSO with the dangers (OBSTRN, WRECKS, UWTROC...) marked by the clearing or transit line.

Figure 16 – Range systems

### 10.1.3			Measured distance

If the track to be followed is on a leading line or a bearing, it is encoded in the way described above (cases 1 or 4). If not, it is encoded only as a NAVLNE with no value for CATNAV. In either case the measured distance is encoded in INFORM.

The transit lines are encoded as NAVLNE with CATNAV = 2.

The beacons are encoded as BCNSPP with CATSPM = 17 .

Each transit line with its beacons must be aggregated into a collection object C_AGGR. These two aggregation objects and the track to be followed must be aggregated into another collection object C_AGGR.

## 10.2			Routeing measures

### 10.2.1			Traffic separation scheme

Traffic separation schemes are encoded by means of DWRTCL, DWRTPT,  ISTZNE, PRCARE, TSELNE, TSEZNE, TSSBND, TSSCRS, TSSLPT, TSSRON and navigational marks.

Relationships between these objects are defined in clause 10.2.3.

#### 10.2.1.1	    	Traffic separation scheme lanes (see M4 - §435.1)

A complete traffic separation scheme lane consists of one or more areas within which the flow of traffic follows one defined direction. Each of these areas is encoded as a TSSLPT.

Geo object:		Traffic separation scheme lane part (TSSLPT)

Attributes:		CATTSS		DATEND		DATSTA

ORIENT -	direction of the traffic flow

RESTRN		STATUS		INFORM		NINFOM

Remarks:

- In junctions, other than crossings and roundabouts, the attribute ORIENT is omitted to avoid implying that one lane has priority over another (see INT1 IM22). INFORM (or TXTDSC) may carry a warning text.

Figure 17 - Junction

#### 10.2.1.2		Traffic separation scheme boundaries (see M4 - §435.1)

Traffic separation scheme boundaries are used to encode the outer limits of traffic lanes or traffic separation scheme roundabouts.

Geo object:		Traffic separation scheme boundary (TSSBND)

Attributes:		CATTSS		DATEND		DATSTA		STATUS		INFORM		NINFOM

#### 10.2.1.3		Traffic separation lines (see M4 - §435.1)

Traffic separation lines are used to separate two traffic lanes or one traffic lane and one inshore traffic zone.

Geo object:		Traffic separation line (TSELNE)

Attributes:		CATTSS		DATEND		DATSTA		STATUS		INFORM		NINFOM

#### 10.2.1.4		Traffic separation zones (see M4 - §435.1)

Traffic separation zones are used to separate two traffic lanes or one traffic lane and one inshore traffic zone or to encode the centre part of a roundabout.

Geo object:		Traffic separation zone (TSEZNE)

Attributes:		CATTSS		DATEND		DATSTA		STATUS		INFORM		NINFOM

#### 10.2.1.5		Traffic separation scheme crossings (see M4 - §435.1)

- Traffic separation scheme crossings are used to encode a crossing between at least four traffic lanes.

Geo object:		Traffic separation scheme crossing (TSSCRS)

Attributes:		CATTSS		DATEND		DATSTA		RESTRN		STATUS		INFORM		NINFOM

Remarks:

- Junctions other than crossings and roundabouts are encoded as TSSLPT (see clauses 10.2.1.1 and 10.2.1.6).

- The object TSSCRS does not overlap the TSEZNE at its centre.

#### 10.2.1.6		Traffic separation scheme roundabouts (see M4 - §435.1)

Geo object:		Traffic separation scheme roundabout (TSSRON)

Attributes:		CATTSS		DATEND		DATSTA		RESTRN		STATUS		INFORM		NINFOM

Remarks:

- The object TSSRON does not overlap the TSEZNE at its centre.

#### 10.2.1.7		Inshore traffic zones (see M4 - §435.1)

Geo object:		Inshore traffic zone (ISTZNE)

Attributes:		CATTSS		DATEND		DATSTA		RESTRN		STATUS		INFORM		NINFOM

#### 10.2.1.8		Precautionary areas (see M4 - §435.2)

Geo object:		Precautionary areas (PRCARE)

Attributes:		DATEND		DATSTA		RESTRN		STATUS		INFORM		NINFOM

Remarks:

- A PRCARE may overlap with other elements of the traffic separation scheme (TSSRON, TSSLPT, TSSCRS...).

### 10.2.2			Deep water routes (see M4 - §435.3)

#### 10.2.2.1		Deep water route parts

A complete deep water route consists of one or more areas within which the flow of traffic either follows one defined direction for one-way traffic or one defined direction and its reciprocal if the traffic is two-way. Each of these area is encoded as a DWRTPT.

Geo object:		Deep water route part (DWRTPT)

Attributes:		DATEND		DATSTA

DRVAL1 -	minimum depth

DRVAL2

NOBJNM

OBJNAM -	only used if the individual object is not aggregated in a collection object

ORIENT -	direction of the traffic flow

QUASOU	RESTRN		SOUACC	STATUS		TECSOU		TRAFIC		VERDAT

INFORM		NINFOM

Remarks:

- This route is covered by objects DEPARE.

- A deep water route part may overlap with a TSSLPT.

#### 10.2.2.2		Deep water route centrelines

Geo object:		Deep water route centreline (DWRTCL)

Attributes:		CATTRK		DATEND		DATSTA

DRVAL1 -	minimum depth

DRVAL2

NOBJNM

OBJNAM -	only used if the individual object is not aggregated in a collection object

ORIENT		QUASOU	SOUACC	STATUS		TECSOU		TRAFIC		VERDAT

INFORM		NINFOM

Remarks:

- When the traffic flow is one way, the direction of digitising should be the same direction as the traffic flow.

### 10.2.3			Traffic separation scheme systems

To encode a traffic separation scheme system the objects DWRTCL, DWRTPT, ISTZNE, PRCARE, TSELNE, TSEZNE, TSSBND, TSSCRS, TSSLPT, TSSRON and navigational aids, if they are stated in the regulation defining the TSS or DW, must be aggregated in a collection object C_AGGR.

Collection object:		Aggregation (C_AGGR)

Attributes:				NOBJNM

OBJNAM -	name of the TSS or DW

INFORM -	textual information about the whole TSS or DW

NINFOM

Remarks:

- A collection object C_AGGR may be used in the same way to encode a whole deep water route.

### 10.2.4			Recommended routes (see M4 - §435.4)

Geo object:		Recommended route centreline (RCRTCL)

Attributes:		CATTRK		DATEND		DATSTA

DRVAL1 -	minimum depth

DRVAL2		NOBJNM		OBJNAM		ORIENT		PEREND		PERSTA		QUASOU

SOUACC	STATUS		TECSOU		TRAFIC		VERDAT		INFORM		NINFOM

Remarks:

- When the traffic flow is one way, the direction of digitising should be the same direction as the traffic flow.

### 10.2.5			Recommended direction of traffic flow (see M4 - §435.5)

The object class RCTLPT is used to encode areas with a recommended traffic flow :

- between two TSS (INT1 - IM26.1)

- in the entrance area of a TSS

- along the outside of a deep water route (INT1 - IM26.2)

Geo object:		Recommended traffic lane part (RCTLPT)

Attributes:		DATEND		DATSTA		ORIENT		STATUS		INFORM		NINFOM

Remarks:

- When the area is not defined, a point object may be encoded.

### 10.2.6			Two-way routes (see M4 - §435.6)

A two-way route is divided into areas within which the flow of traffic flows in two directions along one bearing. Each of these areas is encoded as a TWRTPT. These route parts will generally be two-way but some may be restricted to one-way traffic flow (INT1 - IM28.2).

Geo object:		Two-way route part (TWRTPT)

Attributes:		CATTRK		DATEND		DATSTA

DRVAL1 -	minimum depth

DRVAL2		ORIENT		QUASOU	SOUACC	STATUS		TECSOU		TRAFIC

VERDAT		INFORM		NINFOM

Figure 18 – One-way traffic flow in a two-way route

In a two-way route with one-way sections, separate area objects should be made for parts with TRAFIC = 3 (one-way) and TRAFIC = 4 (two-way). In one-way sections the attribute ORIENT must indicate the true direction of traffic flow, not its reciprocal. In two way sections ORIENT may indicate either direction.

### 10.2.7			Areas to be avoided (see M4 - §435.7)

Areas to be avoided are encoded as RESARE with RESTRN = 14. An area to be avoided around a navigational aid is encoded with CATREA = 12.

## 10.3			Ferries (see M4 - §438)

Geo object:		Ferry route (FERYRT)

Attributes:		CATFRY		DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA

STATUS		INFORM		NINFOM

## 10.4			Fairways

Geo object:		Fairway (FAIRWY)

Attributes:		DATEND		DATSTA

DRVAL1 -	minimum depth

NOBJNM		OBJNAM		ORIENT		QUASOU	RESTRN		SOUACC	STATUS

TRAFIC		VERDAT		INFORM		NINFOM

Remarks:

- A collection object C_AGGR or C_ASSO may be created to relate the fairway with the navigational aids, the recommended tracks, the dredged areas, and the other regulations.

- Where beacons or buoys marking a fairway are offset from the actual fairway limits, this will be indicated by the use of INFORM.

# 11.			Regulated areas

## 11.1			Restricted areas in general (see M4 - §439.2 to § 439.4)

Restricted areas are encoded by RESARE or other objects having the attribute RESTRN (ACHARE, CBLARE, DMPGRD, DRGARE, DWRTPT, FAIRWY, ICNARE, ISTZNE, MARCUL, MIPARE, OSPARE, PIPARE, PRCARE, SPLARE, SUBTLN, TESARE, TSSCRS, TSSLPT, TSSRON).

Use of objects of class restricted area:

Geo object:		Restricted area (RESARE)

Attributes:		CATREA -	describes the reason for the regulation

DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA

RESTRN - 	describes the restrictions

STATUS

INFORM -	a short explanation about the regulation (e.g. caution note from the paper chart). (TXTDSC may be used instead).

- NINFOM

Remarks:

- An area where the mariner has to be aware of circumstances influencing the safety of navigation (e.g. an area of continually changing depths, ...) is encoded using the object class CTNARE (see clause 6.6). This object class may be required to identify a danger, a risk, a rule or advice which is not directly related to a particular object.

## 11.2			Maritime jurisdiction areas (see M4 - §440)

Figure 19 – Maritime jurisdiction areas

### 11.2.1			National territory

Geo object:		Administration area (ADMARE)

Attributes:		JRSDTN - 	2 - national

NATION		NOBJNM		OBJNAM		INFORM		NINFOM

### 11.2.2			Custom zones

Geo object:		Custom zone (CUSZNE)

Attributes:		NATION		INFORM		NINFOM

### 11.2.3			Free port area

Geo object:		Free port area (FRPARE)

Attributes:		NOBJNM		OBJNAM		STATUS		INFORM		NINFOM

### 11.2.4			Territorial sea

The territorial sea is limited by:

- territorial sea baselines (drying line),

- straight territorial sea baselines,

- international maritime boundaries,

- seaward limits of territorial seas.

The straight territorial sea baseline is encoded by:

Geo object:		Straight territorial sea baseline (STSLNE)

Attributes:		NATION		INFORM		NINFOM

The territorial sea area is encoded by:

Geo object:		Territorial sea area (TESARE)

Attributes:		NATION		RESTRN	I	NFORM		NINFOM

### 11.2.5			Contiguous zone

Geo object:		Contiguous zone (CONZNE)

Attributes:		DATEND		DATSTA		NATION		STATUS		INFORM		NINFOM

### 11.2.6			Fishery zones

Geo object:		Fishery zone (FSHZNE)

Attributes:		NATION		NOBJNM		OBJNAM		STATUS

INFORM -	value and unit of the associated limit (generally 6 M or 12 M).

- NINFOM

### 11.2.7			Continental shelf

Geo object:		Continental shelf area (COSARE)

Attributes:		NATION		NOBJNM		OBJNAM		INFORM		NINFOM

### 11.2.8			Exclusive economic zone

Geo object:		Exclusive economic zone (EXEZNE)

Attributes:		NATION		INFORM		NINFOM

## 11.3			Military practice areas; minefields

### 11.3.1			Military practice areas (see M4 - §441)

Geo object:		Military practice areas (MIPARE)

Attributes:		CATMPA -	2 - torpedo exercise area

3 - submarine exercise area

4 - firing danger area

5 - mine-laying practice area

6 - small arms firing range

DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA		RESTRN

STATUS		INFORM		NINFOM

### 11.3.2			Submarine transit lanes (see M4 - §441.5)

Geo object:		Submarine transit lane (SUBTLN)

Attributes:		NOBJNM		OBJNAM		INFORM		NINFOM		RESTRN

### 11.3.3			Minefields (see M4 - §441.8)

Minefields are encoded as RESARE with CATREA = 14.

Former mined areas are encoded with STATUS = 4.

## 11.4			Dumping grounds (see M4 - §442)

Geo object:		Dumping ground (DMPGRD)

Attributes:		CATDPG -	2 - chemical waste dumping ground

3 - nuclear waste dumping ground

4 - explosives dumping ground

5 - spoil ground

6 - vessels dumping ground

NOBJNM		OBJNAM		RESTRN		STATUS		INFORM		NINFOM

## 11.5			Cables and cable areas

### 11.5.1			Submarine cables (see M4 - §443)

Geo object:		Cable, submarine (CBLSUB)

Attributes:		BURDEP -	if the buried depth varies along the cable, the cable must be encoded as several objects.

CATCBL -	1 - power line

4 - telephone

5 - telegraph

6 - mooring cable/chain

CONDTN-	1 - under construction (during laying)

5 - planned construction (planned laying)

DATEND		DATSTA		DRVAL1		DRVAL2		NOBJNM		OBJNAM

STATUS -	4 - not in use (disused)

VERDAT		INFORM		NINFOM

Remarks:

- When a cable is disused there is no need to encode the attribute CATCBL.

### 11.5.2			Overhead cables (see M4 - §382)

Geo object:		Cable overhead (CBLOHD)

Attributes:		CATCBL

CONDTN-	1 - under construction

5 - planned construction

CONRAD	CONVIS		DATEND		DATSTA		ICEFAC		NOBJNM		OBJNAM

STATUS

VERACC -	applies to VERCLR or VERCSA

VERCLR -	vertical clearance over VERDAT

VERCSA -	safe vertical clearance over VERDAT

VERDAT		INFORM		NINFOM

Remarks:

- Telepheric cables are encoded as CONVYR (see clause 4.8.11) with CATCON = 1.

- When a cable has radar reflectors, they must be encoded as separate objects RADRFL. If the whole cable is radar conspicuous or the compilation scale is too small to show individual reflectors, it is encoded with CONRAD.

### 11.5.3			Submarine cable areas (see M4 - §443.2)

Geo object:		Cable area (CBLARE)

Attributes:		CATCBL -	1 - power line

4 - telephone

5 - telegraph

DATEND		DATSTA		NOBJNM		OBJNAM		RESTRN

STATUS -	this attribute encodes the status of the area and not the status of the cables in this area.

INFORM		NINFOM

## 11.6			Pipelines and pipeline areas

### 11.6.1			Pipelines, submarine or on land (see M4 - §377 and §444)

Geo object:		Pipeline, submarine / on land (PIPSOL)

Attributes:		BURDEP -	if the buried depth varies along the pipeline, it must be encoded as several objects.

CATPIP -	2 - outfall

3 - intake

4 - sewer

5 - bubbler system

6 - supply pipe

CONDTN -	1 - under construction (during laying)

5 - planned construction (planned laying)

DATEND		DATSTA		DRVAL1		DRVAL2		NOBJNM		OBJNAM		PRODCT

STATUS -	4 - not in use (disused)

INFORM		NINFOM		VERACC		VERDAT		VERLEN

Remarks:

- DRVAL1 and DRVAL2 may be used to encode the least and maximum depths over the pipe.

- When a pipeline is disused there is no need to encode the attributes CATPIP and PRODCT.

### 11.6.2			Diffusers, cribs

Diffusers and cribs are encoded as OBSTRN (see clause 6.2.2) with:

CATOBS -	3 - diffuser

4 - crib

### 11.6.3			Overhead pipelines (see M4 - §383)

Geo object:		Pipeline overhead (PIPOHD)

Attributes:		CATPIP

CONDTN -	1 - under construction

5 - planned construction

CONRAD	CONVIS		DATEND		DATSTA		NOBJNM		OBJNAM		PRODCT

STATUS

VERACC -	applies to VERCLR

VERCLR -	vertical clearance over VERDAT

VERDAT		INFORM		NINFOM

- When an overhead pipeline is disused there is no need to encode the attributes CATPIP and PRODCT.

### 11.6.4			Pipeline areas (see M4 - §444.2)

Geo object:		Pipeline area (PIPARE)

Attributes:		CATPIP		CONDTN	DATEND		DATSTA		NOBJNM		OBJNAM		PRODCT

RESTRN

STATUS -	this attribute encodes the status of the area and not the status of the pipelines in this area.

INFORM		NINFOM

## 11.7			Oil and Gas fields

### 11.7.1			Wellheads (see M4 - §445.1, §445.5 and §445.7)

Wellheads are encoded as OBSTRN (see clause 6.2.2) with:

CATOBS -	2 - wellhead

STATUS -	4 - not in use (disused)

VERLEN -	vertical length above the bottom

WATLEV-	2 - always dry (for wellheads which protrude at high water)

3 - always under water/submerged

### 11.7.2			Offshore platforms (see M4 - §445.2)

Geo object:		Offshore platform (OFSPLF)

Attributes:		CATOFP		COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND

DATSTA

HEIGHT -	 for fixed platforms, referred to the vertical datum

NATCON		NOBJNM		OBJNAM		PRODCT		STATUS		VERACC		VERDAT

VERLEN -	 for floating platforms, referred to the sea level

INFORM		NINFOM

Remarks:

- Sites of dismantled platforms are encoded as OBSTRN (see clause 6.2.2) with CATOBS = 7 (foul ground).

- Platforms may carry lights (see clause 12.8), fog signals (see clause 12.5) and helicopter platforms (see clause 4.8.12).

### 11.7.3			Safety zones (see M4 - §445.2)

- Safety zones are encoded as RESARE with CATREA = 1.

### 11.7.4			Offshore production area (see M4 - §445.3)

Geo object:		Offshore production area (OSPARE)

Attributes:		CATPRA		CONDTN	CONRAD	CONVIS		DATEND		DATSTA		HEIGHT

INFORM		NINFOM		NOBJNM		OBJNAM		PRODCT		RESTRN		STATUS

VERACC		VERLEN

### 11.7.5			Offshore tanker loading systems (see M4 - §445.4)

- Very large tanker loading buoys are encoded as BOYINB .

Articulated towers are encoded as OFSPLF with:

CATOFP -	4 - articulated loading platform (ALP)

5 - single anchor leg mooring (SALM)

8 - floating production, storage and off-loading vessel (FPSO)

10 - navigation, communication and control buoy (NCCB) (which may include

storage facilities)

### 11.7.6			Flare stacks (see M4 - §445.6)

Flare stacks are encoded as LNDMRK (see clause 4.8.15) with CATLMK = 6.

An offshore flare stack may share the same geometry as the platform on which it is installed.

## 11.8			Spoil grounds, dredging areas (see M4 - §446)

Spoil grounds are encoded as DMPGRD (see clause 11.4) with CATDPG = 5.

Dredging areas are encoded as RESARE with CATREA = 21.

## 11.9			Fishing equipment and aquaculture areas

### 11.9.1			Fishing facilities (see M4 - §447.1 to §447.3)

Geo object:		Fishing facilities (FSHFAC)

Attributes:		CATFIF -	1 - fishing stake

2 - fish trap

3 - fish weir

4 - tunny net

NOBJNM		OBJNAM		PEREND		PERSTA		STATUS		VERACC

VERLEN -	height above sea bottom

INFORM		NINFOM

### 11.9.2			Marine farms (see M4 - § 447.4 and §447.6)

Geo object:		Marine farms / culture (MARCUL)

Attributes:		CATMFA -	1 - crustaceans

2 - oysters / mussels

3 - fish

4 - seaweed

5 – pearl culture farm

DATEND		DATSTA		EXPSOU		INFORM		NINFOM		NOBJNM		OBJNAM

PEREND		PERSTA		QUASOU	RESTRN		SOUACC	STATUS		VALSOU

VERDAT

VERLEN -	height above sea bottom

WATLEV

- The attributes EXPSOU, QUASOU, VALSOU encode the minimum depth of the object.

### 11.9.3			Fish havens (see M4 - §447.5)

- Fish havens are encoded as OBSTRN (see clause 6.2.2) with CATOBS = 5.

### 11.9.4			Fishing ground

- A place where fishing is frequently carried out is encoded as a FSHGRD.

Geo object:		Fishing ground (FSHGRD)

Attributes:		NOBJNM		OBJNAM		PEREND		PERSTA		STATUS		INFORM		NINFOM

## 11.10		Degaussing ranges (see M4 - §448)

- Degaussing ranges are encoded as RESARE (see clause 11.1) with CATREA = 8.

## 11.11		Historic wrecks (see M4 §449.5)

Restricted areas around historic wrecks are encoded as RESARE (see clause 11.1) with CATREA = 10.

In addition the wreck may be encoded as a wreck object (see clause 6.2.1).

## 11.12		Seaplane landing area (see M4 §449.6)

Geo object:		Seaplane landing area (SPLARE)

Attributes:		NOBJNM		OBJNAM		PEREND		PERSTA		RESTRN		STATUS		INFORM

NINFOM

Remarks:

- An anchorage for seaplanes is encoded as ACHARE (see clause 9.2.1) with CATACH = 6.

## 11.13		Various maritime areas

### 11.13.1		Ice areas (see M4 - §449.1)

Geo object:		Ice area (ICEARE)

Attributes:		CATICE		CONVIS		ELEVAT		HEIGHT		NOBJNM		OBJNAM		PEREND

PERSTA		STATUS		VERACC		VERDAT		VERLEN		INFORM		NINFOM

### 11.13.2		Log ponds (see M4 - §449.2)

Geo object:		Log ponds (LOGPON)

Attributes:		NOBJNM		OBJNAM		STATUS		INFORM		NINFOM

### 11.13.3		Incineration area (see M4 - §449.3)

Geo object:		Incineration area (ICNARE)

Attributes:		NOBJNM		OBJNAM		PEREND		PERSTA		RESTRN		STATUS		INFORM

NINFOM

### 11.13.4		Cargo transhipment area (see M4 - §449.4)

Geo object:		Cargo transhipment area (CTSARE)

Attributes:		DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA		STATUS

INFORM		NINFOM

## 11.14 		Nature reserves

Marine nature reserves should be encoded as RESARE (see clause 11.1) with CATREA = 4. Nature reserves that are shown on the source as point symbols should be encoded point CTNARE with ‘Marine nature reserve’ encoded in INFORM.

# 12.			Aids to navigation

## 12.1			Lighthouses, navigational marks - Relationship

### 12.1.1			Geo objects part of navigational aids

- Aids to navigation are composed of fixed or floating structures carrying equipment objects.

The most common structure objects are: BCNCAR, BCNISD, BCNLAT, BCNSAW, BCNSPP, BOYCAR, BOYINB, BOYISD, BOYLAT, BOYSAW, BOYSPP, BRIDGE, BUISGL, DAYMAR, LITFLT, LITVES, LNDMRK, MORFAC, OFSPLF, PILPNT, SLCONS.

Equipment objects: DAYMAR, FOGSIG, LIGHTS, RADSTA, RDOSTA, RETRFL, RTPBCN, SISTAT, SISTAW, TOPMAR.

Radar reflectors are not encoded as separate objects when attached to navigational aids. Their existence is indicated by CONRAD = 3 for the structure object.

Rescue stations and coastguard stations are not related directly to navigation, so they are not part of the equipment objects of navigational aids. If they are installed at the same place as a navigational mark, they share the same spatial object.

### 12.1.2			Relationships

A master to slave relationship must be created to relate the different objects composing the navigational aid.

When the navigational aid contains a structure object (from the above list), this object must be the master object, the equipment objects are the slaves.

When the nature of the base structure is unknown or there is no structure object, one of the equipment objects is chosen as the master, giving the priority to one of the LIGHTS if one exists.

Figure 20  – Navigation aids

The name of the navigational aid must be encoded in the attribute OBJNAM and possibly NOBJNM of the master object. It is not repeated for each slave object. If the name is painted on the structure, it must be encoded with the same spelling in OBJNAM if it is a Latin alphabet, or in NOBJNM if it is not.

- All point objects composing a navigational aid must point to the same point spatial object.

The navigational aid may be associated in a C_ASSO with the objects which it marks (restricted area, obstruction...). Several navigational aids and several marked objects may be associated in the same relationship.

## 12.2			Buoyage system and direction of buoyage (see M4 - §461)

The buoyage system and, where necessary, the direction of buoyage are encoded with the meta object M_NSYS:

Meta object:		Navigational system of marks (M_NSYS)

Attributes:		MARSYS		ORIENT		INFORM		NINFOM

All parts of the data set must be covered by M_NSYS meta objects with the attribute MARSYS to indicate the buoyage system in operation. M_NSYS objects with the attribute MARSYS must not overlap.

Within a dataset there may be some areas where the direction of buoyage is defined by local rules and must, therefore, be specified. These areas should be encoded as M_NSYS areas with the attribute ORIENT to indicate the direction of buoyage. M_NSYS objects with the attribute ORIENT must not overlap but in areas where local buoyage directions apply M_NSYS area objects  with the attribute ORIENT will overlap  M_NSYS area objects with attribute MARSYS.

Individual buoys and beacons may not be part of the general buoyage system. This is encoded in MARSYS.

Figure 21 – Buoyage system and direction

## 12.3			Fixed structures (see M4 - §455)

### 12.3.1			Beacons

Geo objects:		Beacon, cardinal (BCNCAR)

Beacon, isolated danger (BCNISD)

Beacon, lateral (BCNLAT)

Beacon, safe water (BCNSAW)

Beacon, special purpose (BCNSPP)

Attributes:		BCNSHP

CATCAM -	only for BCNCAR

CATLAM -	only for BCNLAT

CATSPM -	only for BCNSPP

COLOUR	COLPAT		CONDTN	CONRAD	CONVIS		DATEND		DATSTA

ELEVAT		HEIGHT

MARSYS -	only if different to the value encoded in M_NSYS

NATCON		NOBJNM		OBJNAM		PEREND		PERSTA

STATUS		VERACC		VERDAT		VERLEN		INFORM		NINFOM

Remarks:

- ELEVAT may encode the altitude of the ground level, but only for beacons built on land.

- HEIGHT encodes the total altitude of the beacon, including the equipment objects (topmark, light...), over the vertical datum.

- VERLEN encodes the total vertical length of the beacon, including the equipment objects (topmark, light...), above the seabed or ground.

- If a cairn bears the colour(s) specified by a navigational mark system, it must be encoded as a beacon.

- If a beacon has more than one colour, the attribute COLOUR should be encoded according to the rules laid out in clause 2.4.

In the following table the symbol ‘/’ means that this attribute is not relevant. A blank means that the encoder can choose an appropriate value.

| Feature | INT1 | Object-class | BCNSHP | CATSPM | Other attributes |
| --- | --- | --- | --- | --- | --- |
| minor not permanent mark | IQ90 | BCN*** | 1 |  |  |
|  | IQ91 | BCNLAT | 1 | / |  |
|  | IQ92 | BCNLAT | 2 | / |  |
| Cairn | IQ100 | BCN*** | 6 |  |  |
| coloured or white mark | IQ101 | DAYMAR | / |  | NATCON = 9 |
| coloured topmark with function of beacon | IQ102.1 | DAYMAR | / |  | NATCON = 9 |
| painted board with function of leading beacon | IQ102.2 | DAYMAR | / | 16 | NATCON = 9, TOPSHP = 6 |
| beacon tower | IQ110 | BCN*** | 3 |  |  |
| lattice beacon | IQ111 | BCN*** | 4 |  |  |
| leading beacon | IQ120 | BCNSPP |  | 16 |  |
| beacon marking a clearing line | IQ121 | BCNSPP |  | 41 |  |
| beacon marking measured distance | IQ122 | BCNSPP |  | 17 |  |
| cable landing beacon | IQ123 | BCNSPP |  | 6 |  |
| outfall landing beacon | IQ123 | BCNSPP |  | 8 |  |
| pipeline landing beacon | IQ123 | BCNSPP |  | 39 |  |
| refuge beacon | IQ124 | BCNSPP |  | 44 |  |
| firing practice area beacon | IQ125 | BCNSPP |  | 1 |  |
| notice board | IQ126 | BCNSPP |  | 18 |  |
| buoyant beacon | IP5 | BCN*** | 7 |  |  |
|  |  |  |  |  | table 12.1 |

### 12.3.2			Lighthouses (see M4 - § 457.3)

Lighthouses are encoded as LNDMRK with CATLMK = 17 and FUNCTN = 33 for towers, or as BUISGL with FUNCTN = 33 for other shapes.

The attributes ELEVAT, HEIGHT and VERLEN are encoded in the same way as for the beacons.

### 12.3.3			Daymarks

Geo object:		Daymark (DAYMAR)

Attributes:		CATSPM - 	16 - leading mark

COLOUR	COLPAT		DATEND		DATSTA		ELEVAT		HEIGHT		NATCON

NOBJNM		OBJNAM		PEREND		PERSTA		STATUS		TOPSHP		VERACC

VERDAT		VERLEN		INFORM		NINFOM

Remarks:

- The attributes ELEVAT, HEIGHT and VERLEN are encode in the same way as for the beacons (see clause 12.3.1).

## 12.4			Floating structures (see M4 - §460)

### 12.4.1			Buoys

Geo objects:		Buoy, cardinal (BOYCAR)

Buoy, installation (BOYINB)

Buoy, isolated danger (BOYISD)

Buoy, lateral (BOYLAT)

Buoy, safe water (BOYSAW)

Buoy, special purpose (BOYSPP)

Attributes:		BOYSHP

CATCAM -	only for BOYCAR

CATINB -	only for BOYINB

CATLAM -	only for BOYLAT

CATSPM -	only for BOYSPP

COLOUR	COLPAT		CONRAD	DATEND		DATSTA

MARSYS -	only if different to the value encoded in M_NSYS

NATCON		NOBJNM		OBJNAM		PEREND		PERSTA

PRODCT -	only for BOYINB

STATUS		VERACC		VERLEN		INFORM		NINFOM

Remarks:

- VERLEN may encode the total vertical length of the buoys, including the equipment objects, above water level.

- If a buoy has more than one colour, the attribute COLOUR should be encoded according to the rules laid out in clause 2.4.

In the following table the symbol ‘/’ means that this attribute is not relevant. A blank means that the encoder may chose a value.

| Feature | INT1 | Object-class | BOYSHP | CATSPM | Other attributes |
| --- | --- | --- | --- | --- | --- |
| mooring buoy | IQ40 | MORFAC |  | / | CATMOR = 7 |
| oil or gas installation buoy | IL16 | BOYINB | 7 | / | PRODCT = 1 or 2, CATINB = 1 or 2 |
| firing danger area buoy | IQ50 | BOYSPP |  | 1 |  |
| Target | IQ51 | BOYSPP |  | 2 |  |
| marker ship | IQ52 | BOYSPP |  | 3 |  |
| Barge | IQ53 | BOYSPP |  | 5 |  |
| degaussing range buoy | IQ54 | BOYSPP |  | 4 |  |
| buoy marking cable | IQ55 | BOYSPP |  | 6 |  |
| spoil ground buoy | IQ56 | BOYSPP |  | 7 |  |
| buoy marking outfall | IQ57 | BOYSPP |  | 8 |  |
| Feature | INT1 | Object-class | BOYSHP | CATSPM | Other attributes |
| buoy marking pipeline |  | BOYSPP |  | 39 |  |
| Superbuoy | IQ26 | BOY*** | 7 |  |  |
| large automatic navigational buoy | IP6 | BOYSPP | 7 | 15 |  |
| data-collecting buoy of superbuoy size | IQ58 | BOYSPP | 7 | 9 |  |
| buoy marking wave recorder (or current meter) | IQ59 | BOYSPP |  | 10 | INFORM = wave recorder (e.g.) |
| seaplane anchorage buoy | IQ60 | BOYSPP |  | 11 |  |
| buoy marking traffic separation scheme | IQ61 | BOYSPP |  | 19 |  |
| buoy marking recreation zone | IQ62 | BOYSPP |  | 12 |  |
|  |  |  |  |  | table 12.2 |

### 12.4.2			Light float and light vessel (see M4 - § 462.8 and §474.1)

Geo objects:		Light float (LITFLT)

Light vessel (LITVES)

Attributes:		COLOUR	COLPAT		CONRAD	CONVIS		DATEND		DATSTA		HORACC

HORLEN		HORWID		NATCON		NOBJNM		OBJNAM		PEREND		PERSTA

STATUS		VERACC		VERLEN		INFORM		NINFOM

## 12.5			Fog signals (see M4 - §451)

Geo object:		Fog signal (FOGSIG)

Attributes:		CATFOG		DATEND		DATSTA		NOBJNM		OBJNAM		SIGFRQ		SIGGEN

SIGGRP		SIGPER		SIGSEQ		STATUS		VALMXR		INFORM		NINFOM

## 12.6			Topmarks (see M4 - §463)

Geo object:		Topmark (TOPMAR)

Attributes:		COLOUR	COLPAT

HEIGHT - 	not used when the topmark is attached to a buoy or to a beacon

MARSYS -	value given by M_NSYS or the structure object

STATUS		TOPSHP		VERACC		VERDAT

VERLEN -	not used when the topmark is attached to a buoy (see clause 12.4.1) or to

a beacon (see clause 12.3.1)

INFORM		NINFOM

## 12.7			Retro-reflectors

Geo object:		Retro-reflector (RETRFL)

Attributes:		COLOUR	COLPAT		HEIGHT

MARSYS -	value given by M_NSYS or the structure object

STATUS		VERACC		VERDAT

INFORM - 	describes letters, patterns or numerals shown.

- NINFOM

## 12.8			Lights (see M4 - §470)

If the actual structure which supports a light in a water area is unknown, a PILPNT should also be encoded at the same position as the light.

12.8.1			Description of lights

Each sector of a light is encoded with one object LIGHTS. They are all slave objects of the same master object which is the structure object or one of the LIGHTS.

Geo object:		Light (LIGHTS)

Attributes:		CATLIT		COLOUR	DATEND		DATSTA 	EXCLIT

HEIGHT-	prohibited for floating lights

LITCHR		LITVIS		MARSYS	MLTYLT		NOBJNM		OBJNAM

ORIENT -	only for directional or moiré effect lights

PEREND		PERSTA

SECTR1 -	only for sector lights

SECTR2 -	only for sector lights

SIGGRP -	prohibited for fixed lights

SIGPER -	prohibited for fixed lights

SIGSEQ		STATUS		VALNMR		VERACC

VERDAT -	applies to HEIGHT ; this value is only used if different to the value encoded in the subfield VDAT or M_VDAT

INFORM		NINFOM

Remarks:

- If it is necessary to encode details of the lighting technology (e.g. neon), the attribute INFORM should be used.

- The purpose of a marine spot light is encoded in the attribute INFORM.

### 12.8.2			Types and functions of lights (see M4 - §471.1)

- Types and functions of lights are encoded with the attribute CATLIT.

### 12.8.3			Rhythms of lights (see M4 - §471.2)

- Rhythms of lights are encoded with the attributes LITCHR and SIGGRP.

### 12.8.4			Elevations of lights (see M4 - §471.6)

The elevation of a light on a fixed structure is encoded with the attribute HEIGHT.

The height above the water surface of a light on a floating structure should be encoded using INFORM.

### 12.8.5			Times of exhibition and exhibition conditions

#### 12.8.5.1	Night lights

Lights that are normally exhibited from about sunset to about sunrise, they are encoded with EXCLIT = 4 (night light).

#### 12.8.5.2	Unwatched lights (see M4 - §473.1)

Generally this information is not encoded, but unwatched lights (unmanned) with no standby or emergency arrangements may be encoded with STATUS = 17.

#### 12.8.5.3	Occasional lights (see M4 - §473.2)

Lights exhibited only when specially needed are encoded with STATUS =2 (occasional light). Private lights are encoded with STATUS = 2,8 (private light, not regularly exhibited).

#### 12.8.5.4	Daytime lights (see M4 - §473.4)

Lights shown throughout the 24 hours without change of character with the possible exception of VALNMR are encoded with EXCLIT = 1.

A light having characteristics shown by day different to those shown at night is encoded as two objects LIGHTS pointing to the same point spatial object:

- one LIGHTS with EXCLIT = 2 (daytime light),

- one LIGHTS with EXCLIT = 4 (night light).

#### 12.8.5.5	Fog lights (see M4 - §473.5)

- Lights exhibited only in fog are encoded as LIGHTS with EXCLIT = 3 and STATUS = 2.

A light having characteristics shown in fog different to those shown in conditions of normal visibility is encoded as two objects LIGHTS with the same point spatial object:

- one LIGHTS with 	EXCLIT = 3 (fog light) and STATUS = 2

- one LIGHTS with 	EXCLIT = 2 (daytime light) or 4 (night light) INFORM = Character of the light changes in fog.

### 12.8.6			Sector lights and lights not visible all round (see M4 - §475)

#### 12.8.6.1	Sector lights (see M4 - §475.1)

Each sector in which the light is visible from seaward is encoded with one object LIGHTS.

No object is created to encode a sector where no light is transmitted.

Limits of sectors are encoded with the attributes SECTR1 and SECTR2.

Figure 22 – Obscured light sectors

#### 12.8.6.2	Lights obscured by obstructions (see M4 - §475.3)

If a light is obscured in a part of the navigable area of a sector (see  Figure 22 - A), beyond an offshore obstruction, it is encoded as several objects LIGHTS. The partially obscured sector (b) seaward of the island, is encoded as a LIGHTS with LITVIS = 8 (partially obscured) and INFORM = Sector obscured only beyond ... The sectors in which the light is visible from seaward (a and c) are encoded in the normal way.

If there is no navigable water between the light and the obstacle (see  Figure 22 - B), the masked sector is encoded as a LIGHTS with LITVIS = 3 (faint, partial obstructions) or 7 (obscured).

#### 12.8.6.3	White fairway sectors (see M4 - §475.5)

The light sectors are encoded in the normal way. The fairway corresponding to the succession of navigable areas in the white sectors may be encoded as a FAIRWY (see clause 10.4).

#### 12.8.6.4	Leading lights (see M4 - §475.6)

- The sector in which the leading light is visible from seaward is encoded as a LIGHTS with

CATLIT =	4,12 - front leading light

4,13 - rear leading light

4,14 - lower leading light

4,15 - upper leading light

Remarks:

- The attribute ORIENT is not used for leading lights, except for directional lights.

- Even if, on the paper chart, the leading lights are merged in only one symbol, at least one object LIGHTS must be created for each of them.

- The leading line must be encoded by the method described in clause 10.1.

#### 12.8.6.5	Directional lights (see M4 - §475.7)

The very narrow sector intended to mark the direction to be followed is encoded as a LIGHTS with CATLIT = 1 (directional function) and, if the light is intensified in this sector this is encoded with LITVIS = 4.

The attribute ORIENT indicates the orientation of the leading line of the directional light measured from seaward.

The recommended track must be encoded by the method described in clause 10.1.

#### 12.8.6.6	Moiré effect lights (see M4 - §475.8)

Moiré effect lights are encoded as LIGHTS with CATLIT = 16.

The attribute ORIENT indicates the orientation of the leading line of the moiré effect light measured from seaward.

The recommended track must be encoded by the method described in clause 10.1.

### 12.8.7			Various special types of lights

| Type | M-4 | CATLIT | Remarks |
| --- | --- | --- | --- |
| subsidiary light | 471.8 | 10 |  |
| aero light | 476.1 | 5 |  |
| air obstruction light | 476.2 | 6 |  |
| fog detector light | 477 | 7 |  |
| bearing light | 478.1 | 18 |  |
| flood light | 478.2 | 8 | Only to code a flood light visible from seaward. The illuminated structure is encoded in the normal way with STATUS = 12. |
| synchronised lights | 478.3 |  | STATUS =15. Synchronised lights may be associated in C_ASSO. |
| strip light | 478.5 | 9 |  |
| spot light |  | 11 | Only to code a spot light visible from seaward. The illuminated feature is encoded in the normal way with STATUS = 12. |
| Emergency light |  | 17 | Encode as a separate object to the main LIGHTS |
| horizontally disposed lights | 471.8 | 19 | The number of lights is encoded using MLTYLT |
| vertically disposed lights | 471.8 | 20 | The number of lights is encoded using MLTYLT |
|  |  |  | table 12.3 |

### 12.8.8 		Light structures

If a fixed light is located in a water area and the structure which actually supports it is unknown, a PILPNT should also be encoded at the same position as the light. This will ensure that a symbol will be shown on ECDIS when the LIGHTS are turned off during day light navigation.

## 12.9			Radio stations (see M4 - §480 to §485)

Geo object:		Radio station (RDOSTA)

Attributes:		CALSGN		CATROS		COMCHA	DATEND		DATSTA		ESTRNG		NOBJNM

OBJNAM		ORIENT		PEREND		PERSTA		SIGFRQ		STATUS		INFORM

NINFOM

Remarks:

- The object RDOSTA is used to encode the technical equipment itself independent of the building or structure where it is installed.

- Further information e.g. transmission characteristic may be encoded using the attribute INFORM (or TXTDSC).

### 12.9.1			Marine and aero-marine radiobeacons (see M4 - §481)

Marine and aero-marine radiobeacons are encoded as RDOSTA with:

CATROS -	1 - circular (omni-directional) marine or aero-marine radiobeacon

2 - directional radiobeacon

3 - rotating-pattern radiobeacon

4 - Consol beacon

ORIENT -	value of the bearing from seaward for directional radiobeacon

Remarks:

- For a directional radiobeacon the bearing line and the recommended track are encoded as described in clause 10.1. Where the bearing line coincides with a leading line defined by lights or other visual features, navigation lines and recommended tracks are not duplicated. The range system must be aggregated using a collection object (C_AGGR).

### 12.9.2			Aeronautical radiobeacons (see M4 - §482)

Aeronautical radiobeacons are encoded as RDOSTA with CATROS = 7 and CALSGN for identification signal.

### 12.9.3			Radio direction-finding stations (see M4 - §483)

Radio direction-finding stations are encoded as RDOSTA with CATROS = 5 and CALSGN for  identification signal.

### 12.9.4			Coast radio stations providing QTG service (see M4 - §484)

- Coast radio stations providing QTG service are encoded as RDOSTA with CATROS = 6.

## 12.10		Radar beacons (see M4 - §486)

Geo object:		Radar transponder beacon (RTPBCN)

Attributes:		CATRTB -	1 - ramark

2 - racon

3 - leading racon

DATEND		DATSTA		NOBJNM		OBJNAM		RADWAL	SECTR1		SECTR2

SIGGRP -	morse identification letter(s)

SIGSEQ		STATUS		VALMXR		INFORM		NINFOM

Remarks:

- The object RTPBCN is used to encode the technical equipment itself independent of the building or structure on which it is located (e.g. beacon, light-vessel, tower ...)

- For leading racons, the bearing line and the recommended track are encoded as described in clause 10.1. Where the bearing line coincides with a leading line defined by lights or other visual features, navigation lines and recommended tracks are not duplicated. The range system must be aggregated using a collection object (C_AGGR).

- The sweep period may be encoded in INFORM.

## 12.11		Radar surveillance systems (see M4 - §487)

Radar surveillance systems may be encoded by means of the object classes RADRNG, RADLNE, and RADSTA.

### 12.11.1		Radar range (see M4 - §487.1)

Geo object:		Radar range (RADRNG)

Attributes:		COMCHA	DATEND		DATSTA		NOBJNM		OBJNAM		STATUS		INFORM

NINFOM

Remarks:

- A RADRNG is of type area.

### 12.11.2		Radar reference lines (see M4 - §487.2)

Geo object:		Radar line (RADLNE)

Attributes:		NOBJNM		OBJNAM

ORIENT -	value of the bearing from seaward

STATUS		INFORM		NINFOM

### 12.11.3		Radar station (see M4 - §487.3)

Geo object:		Radar station (RADSTA)

Attributes:		CATRAS -	1 - radar surveillance station

2 - coast radar station

COMCHA	DATEND	DATSTA

HEIGHT - height of the emitting part of the radar

NOBJNM		OBJNAM		STATUS		VERACC		VALMXR		VERDAT		INFORM

NINFOM

Remarks:

- The object RADSTA is used to encode the technical equipment itself independent of the building or structure where it is installed. This building or structure (e.g. mast, tower, radar dome...) is encoded as BUISGL or LNDMRK.

## 12.12		Radar conspicuous objects (see M4 - §485.2)

The attribute CONRAD (conspicuous, radar) encodes whether or not an object is radar conspicuous:

CONRAD -	1 - radar conspicuous

2 - not radar conspicuous

3 - radar conspicuous (has radar reflector)

Remarks:

- Objects which have no radar reflector, but are radar conspicuous, must be encoded with CONRAD = 1.

- If a point object is radar conspicuous because it is fitted with a radar reflector, the attribute CONRAD with value 3 must be used. The object class RADRFL is only used to encode radar reflectors on line objects (e.g. overhead cables):

Geo object:		Radar reflector (RADRFL)

Attributes:		HEIGHT		STATUS		VERACC		VERDAT		INFORM		NINFOM

## 12.13		Radio reporting (calling-in) points (see M4 - §488)

Geo object:		Radio calling-in point (RDOCAL)

Attributes:		COMCHA	DATEND		DATSTA		NOBJNM

OBJNAM -	e.g. alphanumeric designator

ORIENT -	orientation of the traffic flow at that point

PEREND		PERSTA		STATUS		TRAFIC

INFORM -	notes; for example, if the requirement to report by radio relates to certain classes of vessels only. (TXTDSC may be used instead).

- NINFOM

Remarks:

- Each object RDOCAL carries only one orientation (and possibly its reciprocal). If the same position is used for another orientation (not opposite) of traffic flow a new object must be created.

- Line RDOCAL must be digitised such that the traffic direction requiring the report is to the right.

- To indicate that the bearing and its reciprocal apply to the RDOCAL, one bearing should be encoded in ORIENT and the attribute TRAFIC should be set to 4 (two-way).

# 13.			Marine services and signal stations

## 13.1			Pilot stations (see M4 - §491)

### 13.1.1			Pilot stations ashore(see M4 - §491.3 and 491.4)

- Pilot stations are encoded as BUISGL or LNDMRK with FUNCTN = 11 (pilot office) or 12 (pilot lookout).

### 13.1.2			Pilot boarding places (see M4 - §491.2)

Geo object:		Pilot boarding place (PILBOP)

Attributes:		CATPIL -	1 - boarding by pilot-cruising vessel

2 - boarding by helicopter

3 - pilot comes out from shore

COMCHA	DATEND		DATSTA		NOBJNM		NPLDST		OBJNAM		PEREND

PERSTA		PILDST		STATUS		INFORM		NINFOM

## 13.2			Coastguard stations (see M4 - §492)

Geo object:		Coastguard station (CGUSTA)

Attributes:		DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA		STATUS

INFORM		NINFOM

Remarks:

- This object class is used to describe the function of the coastguard station. The building or tower in which the coastguard station operates is encoded as a BUISGL or LNDMRK.

## 13.3			Rescue stations (see M4 - §493)

Geo object:		Rescue station (RSCSTA)

Attributes:		CATRSC -	1 - rescue station with lifeboat

2 - rescue station with rocket

4 - refuge for shipwrecked mariners

5 - refuge for intertidal area walkers

6 - lifeboat lying at a mooring

7 - aid radio station

8 – first aid equipment

DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA		STATUS

INFORM		NINFOM

Remarks:

- This object class is used to describe the function of the rescue station.

- The building housing the service is encoded as a BUISGL or LNDMRK.

- A refuge beacon is encoded as a BCNSPP with CATSPM = 44 without RSCSTA.

## 13.4			Signal stations (see M4 - §494 to §497)

- Signal stations are encoded either as traffic signal stations or warning signal stations.

Geo object:		Signal station, traffic (SISTAT)

Attributes:		CATSIT -	1 - port control

2 - port entry and departure

3 - international port traffic

4 - berthing

5 - dock

6 - lock

7 - flood barrage

8 - bridge passage

9 - dredging

10 – traffic control light

COMCHA	DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA

STATUS		INFORM		NINFOM

Remarks:

- Bridge lights marking the centre of navigable spans are encoded as LIGHTS.

- This object class is used to describe the function of the signal station.

- The building housing the service is encoded as a BUISGL or LNDMRK.

Geo object:		Signal station, warning (SISTAW)

Attributes:		CATSIW -	1	- danger

2	- maritime obstruction

3	- cable

4	- military practice

5	- distress

6	- weather

7	- storm

8	- ice

9	- time

10	- tide

11	- tidal stream

12	- tide gauge

13	- tide scale

- - diving

- - water level gauge

COMCHA	DATEND		DATSTA		NOBJNM		OBJNAM		PEREND		PERSTA

STATUS		INFORM		NINFOM

Remarks:

- This object class is used to describe the function of the signal station.

- The building housing the service is encoded as a BUISGL or LNDMRK.

# 14.			Geographic names

International and national geographic names are encoded with the attributes OBJNAM and NOBJNM as described in the ENC Product Specification (S-57 Appendix B.1), clause 3.11. The use of the cartographic object $TEXTS is prohibited.

When possible existing objects (BUAARE, RIVERS, navigational marks...) are used to carry this information.

When there is no existing object able to encode the geographic name, a specific object ADMARE, SEAARE or LNDRGN is created (see clauses 4.7.1 and 8 ). In order to minimise the data volume, these objects may use the geometry of existing objects: e.g. a SEAARE may use the geometry of a DEPARE.

# 15.			Collection objects

Collection object:	Aggregation (C_AGGR)

Attributes:			NOBJNM		OBJNAM		INFORM		NINFOM

C_AGGR objects should be used to encode the link between objects that are functionally related and which together form a higher level entity. For example, a navigation line, a recommended track and the defining navigational aids should be linked using C_AGGR to form a range system.

Collection object:	Association (C_ASSO)

##### Attributes:			NOBJNM		OBJNAM		INFORM		NINFOM

C_ASSO objects should be used to encode the association between two or more objects. The association does not create any higher level entity. For example a wreck should be associated to the buoy that marks it.

- The following table shows the collection objects described in this part of the standard.

| Relationships | FFPT field | collection object | comment |
| --- | --- | --- | --- |
| mooring trots | peer * | C_AGGR | see 9.2.5 |
| measured distances | peer * | C_AGGR | see 10.1.3 |
| Traffic Separation Schemes systems | peer * | C_AGGR | see 10.2.3 |
| Navigation lines and tracks | peer * | C_AGGR | see 10.1.2 |
| Navigation lines, tracks and dangers | peer * | C_ASSO | see 10.1.2 ** |
| Synchronised lights | peer * | C_ASSO | see 12.8.7 |
| airfield, airport, (runway, control tower, etc.) | peer * | C_ASSO | see 4.8.12 |
| tide, tidal stream (non-harmonic prediction – time series or harmonic prediction) | peer * | C_ASSO | see 3.2.3, 3.3.3 and 3.3.4 |
| anchorage | peer * | C_ASSO | see 9.2.6 |
| fairway | peer * | C_ASSO/C_AGGR | see 10.4 |
| radar beacon | peer * | C_AGGR | see 12.10 |
|  |  |  | table 15.1 |

*	Relationships using collection objects are assumed to be peer to peer, RIND subfield of collection feature record must be “peer”. (see ENC P.S. 3.9)

**	This C_ASSO is not used to link the individual elements of the navigation lines and tracks but is used to show the relationship between the navigation lines and tracks, and the associated dangers.

Relationships may be hierarchical. A record in the hierarchy is explicitly linked to its direct children and to its parent. It is also implicitly linked to its grandchildren and grandparents. Therefore, in the interest of simplicity and the reduction of data volume, there is no need to explicitly encode the implied relationships.

Page intentionally left blank

ALPHABETICAL INDEX

A

Accuracy	3, 5

Horizontal	5

Vertical	6

ACHARE	50, 51, 59, 66

ACHBRT	50, 51

ADMARE	26, 60, 82

Aids to navigation	67

AIRARE	1, 25

Airfields	25

Airport	25

Altitude of beacons	31, 69

Anchor berths	50

Anchorage

Relationship	51

Anchorages	50

Prohibited	50

Anchoring restricted	51

Aquaculture areas	65

Areas

Administrative harbour	50

Aquaculture	65

Built-up	26

Cable	62

Cargo transhipment	67

Continual change	41

Depth	33, 35, 37, 38

Dredged	39, 40

Exercise	61

Foul	44, 45

Free port	60

Ice	67

Inadequate depth information	41

Incineration	67

Incompletely surveyed	42

Intertidal	49

Land	12

Maritime jurisdiction	60

Military practice	61

Offshore production	65

Pipeline	63, 64

Precautionary	56

Regulated	59

Restricted	59

Sea	49

Seaplane landing	25, 66

Swept	41

To be avoided	58

Unsafe navigation	46

Unsurveyed	41

Wrecks, foul areas and obstructions	44

Areas to be avoided	58

Articulated loading platform (ALP)	65

Articulated towers	65

B

Bank Office	27

Baselines	60, 61

Basins	12, 40

Tidal & non-tidal	17

BCNCAR	67, 69

BCNISD	67, 69

BCNLAT	67, 69, 70

BCNSAW	67, 69

BCNSPP	31, 54, 67, 69, 70, 81

Beacons	69

buoyant	70

clearing line	70

consol	77

lattice	70

leading	70

measured distance	70

radar transponder	78

refuge	81

Berths	15

BERTHS	15, 51

Bollard	18

bottom quality	See Seabed

Boulder on land	29

BOYCAR	67, 71

BOYINB	65, 67, 71

BOYISD	67, 71

BOYLAT	67, 71

BOYSAW	67, 71

BOYSPP	31, 67, 71, 72

Breakers	46

Breakwater (floating)	19

Bridge

lights	81

supports	25

water under	25

BRIDGE	25, 67

Bridges	25

BUAARE	26, 82

Buddhist Temple	27

Buildings

Bank Office	27

Buddhist Temple	27

Bus Station	27

Church	27

Custom Office	15, 27

Dockside	19

Factory	26

Health Office	15, 28

House	28

Landmarks	26

Marabout	28

Mosque	28

Pagoda	28

Pilot Lookout	28, 79

Pilot Office	15, 28, 79

Police Station	28

Post Office	28

Power Station	26, 28

Railway Station	29

Silos	26

Tanks	26

Transit Shed	29

Transit sheds/warehouses	19

Warehouse	29

Built-up areas	26

BUISGL	              15, 19, 23, 25, 26, 27, 28, 29,

67, 70, 79, 80, 81

Buoyage system  See Buoys	51, 71

Direction of buoyage	68

Mooring	51

navigation, communication and control buoy (NCCB)	65

Spoil ground	71

Bus Station	27

C

C_AGGR	51, 53, 54, 57, 59, 77, 78, 82

C_ASSO	11, 12, 25, 51, 53, 59, 68, 76, 82, 83

Cable areas	62

Cables

Areas	62

Disused	62

Overhead	62

Submarine	62

Telepheric	63

Cairn	69, 70

Canal banks	14, 23

Canals	12, 23, 40

Navigable	23

CANALS	23, 37, 40

CANBNK	23, 40

Cargo transhipment areas	67

Cartographic framework	1

Causeways	24

CAUSWY	24

Caution Area	47

Caution note	41, 59

CBLARE	59, 63

CBLOHD	62

CBLSUB	51, 62

CGUSTA	80

Checkpoints	16

CHKPNT	16

Church	27

Dome	27

Spire	27

Tower	27

Clearing line	52

Cliff contour	33

Cliffs	21, 23

Clock Tower	27

COALNE	14, 17, 20, 21, 22, 23, 40, 43, 49

Coast radio stations (QTG service)	77

Coastguard stations	67, 80

Coastline	14

Artificial	14

Cliff	21

Natural	14

Unsurveyed	14

Collection objects	82

Colours	7

Compilation scale	6

CONRAD	67, 79

Contiguous zone	61

Continental shelf	61

Continual change areas	41

Control	13

Control Tower	25, 27, 82

Conveyors	25

CONVYR	25, 63

CONZNE	61

Cooling Tower	27

COSARE	61

Crane

jib	19

Travelling	19

Cranes	19

CRANES	26

Cribs	45, 63

Cross, Calvary	27

CTNARE	8, 15, 41, 42, 47, 59, 67

CTRPNT	13

CTSARE	67

Cupola	27

CURENT	12

Currents	12

Custom Office	15, 27

Custom zones	60

CUSZNE	60

Cuttings and embankments	23

D

DAMCON	24

Dams	24

Danger line	46

Around point	46

Around rock	44

Limiting wreck area or obstructions	46

Unsafe area	46

Dangers	43

Doubtful	47

Data

Bathymetric	3, 4

Bathymetric source	6

Other source	6

Source	6

Data quality	3

Dates	3

Datum

Horizontal	1

Sounding	2, 32

Vertical	2, 13

DAYMAR	31, 67, 70

Daymarks	70

Deep water route	56

Centerline	56

Part	56

Degaussing ranges	66

DEPARE	2, 15, 17, 21, 22, 23, 24, 33,

34, 35, 36, 37, 38, 39, 40, 41, 42, 44,

45, 46, 49, 56, 82

DEPCNT	2, 25, 32, 34, 41, 49

Depth	2, 3, 32, 33, 34, 35, 37, 38

Contours	2, 32

swept	41

Swept	4

Units	41

Unknown	43

Depth areas	33, 35, 37, 38

Adjoining constructions	38

Adjoining dredged areas	38

Adjoining non-navigable water	37

Always dry	38

DRVAL1 & DRVAL2	34

Geo object	33

Geometry	34

linear depth areas	32

Diffusers	63

Dish Aerial	27

DISMAR	13

Distance marks	13

DMPGRD	59, 62, 65

DOCARE	17, 37, 40

Docks	16

Dry	16

Floating	16

Dolphin	18

Dome	27

Dredged areas	39, 40

Adjoining dredged areas	39

Date of dredging	40

depth maintained	40

Spoil grounds	65

DRGARE	2, 17, 21, 22, 23, 33, 34, 40,

41, 42, 59

DRYDOC	16, 19

Drying

Areas	34

Contours	32

Height	34

Rocky areas	32

Dumping grounds	62

Dunes, sand hills	20

DWRTCL	54, 56, 57

DWRTPT	54, 56, 57, 59

DYKCON	24

Dykes	24

E

Eddies	46

ELEVAT	13

Embankment	See Cuttings and embankments

Exclusive economic zone	61

EXEZNE	61

F

Factory	26

Fairway	58

FAIRWY	58, 59, 75

Features

Artificial	23

Natural	20

Fences and walls	30

Ferries	58

FERYRT	58

Firing range	61

Fish havens	45, 66

Fishery zone	61

Fishing equipment	65

Fishing facilities	65

Fishing ground	66

Fixed structures	69

Flagpole	27

Flagstaff	27

Flare Stack	27, 65

Flare stacks	65

Floating production, storage and off-loading vessel (FPSO)	65

Floating structures	71

FLODOC	16, 33, 34, 42

Flood barrages	24

FNCLNE	26, 30

Fog signals	72

FOGSIG	67, 72

FORSTC	30

Fortified structures	30

Foul areas	44, 45

Foul ground	45

Free port area	60

FRPARE	60

FSHFAC	65

FSHGRD	66

FSHZNE	61

G

GATCON	17, 18, 24

Gates	17

Geographic names	82

Glaciers	22

Grain Elevator	28

Gridiron	18

GRIDRN	18

Ground tackle	45, 51

Groynes	15

H

Harbour

Administrative areas	50

Facilities	15

Installations	15

Offices	15

Regulations	50

Regulations within limits	50

Health Office	15, 28

Height

Contours, spot heights	20

Units	3

Heights and elevations	13

House	28

HRBARE	50

HRBFAC	15

HULKES	19, 33, 34, 42

Hulks	19

I

Ice areas	67

ICEARE	22, 67

ICNARE	59, 67

Inadequate depth information areas	41

Incineration areas	67

Incompletely surveyed areas	42

International maritime boundaries	60

Intertidal Areas	49

ISTZNE	54, 56, 57, 59

J

Junction cables	51

L

LAKARE	22, 40

Lakes	12, 22, 40

LAKSHR	22, 40

Land area	12

Landing place	15

Landmarks

Cairn	69, 70

Church Dome	27

Church Spire	27

Church Tower	27

Clock Tower	27

Control Tower	25, 27, 82

Cooling Tower	27

Cross, Calvary	27

Cupola	27

Dish Aerial	27

Dome	27

Flagstaff	27

Lookout Station	28

Lookout Tower	28

Mast	27, 28, 44, 45, 79

Memorial Plaque	28

Microwave Tower	28

Minaret	28

Monument	28

Mooring Mast	28

Obelisk	28

Observation Tower	28

Radar Dome	28, 79

Radar Tower	28

Radio

Tower	29

Radio Mast	28

Shinto Shrine	29

Spire	29

Television Mast	29

Television Tower	29

Timeball Tower	29

Tower	27, 28, 29, 70, 78, 79, 80

Tree	22

Windmill	29

Landmarks:	27, 28, 65

Lava	22

Light float	72

Light vessel	72

Lighthouses	67, 70

Lights	73, 74, 75

Aero	76

Air obstruction	76

Bearing	76

Daytime	74

Description	73

Directional	76

Elevation	73

Emergency	76

Flood	76

Fog	74

Fog detector	76

Horizontally disposed	76

Leading	75

Moiré effect	76

Night	73

Obscured by obstructions	75

Obscured sectors	74

Occasional	74

Rhythm	73

Sector	74

Special	76

Spot	76

Strip	76

Structures	76

Time of exhibition	73

Types and functions	73

Unwatched	74

White fairway sectors	75

LIGHTS	67, 68, 73, 74, 75, 76, 81

Lines

Leading	52

Navigation	52, 53, 77, 78, 83

Recommended track	52

Transit	52

LITFLT	67, 72

LITVES	67, 72

LNDARE	12, 14, 15, 16, 18, 19, 21, 22,

23, 24, 30, 33, 34, 37, 40, 42, 43

LNDELV	20, 21, 43, 45

LNDMRK	25, 26, 27, 28, 29, 65, 67,

70, 79, 80, 81

LNDRGN	12, 20, 22, 49, 82

Locks	17

LOCMAG	10

Log ponds	67

LOGPON	67

LOKBSN	17, 18, 19, 40

Lookout Station	28

Lookout Tower	28

M

M_ACCY	5

M_CSCL	6

M_HDAT	1

M_HOPA	2

M_NPUB	8

M_NSYS	31, 68, 69, 71, 72

M_PROD	3

M_QUAL	2, 4, 5, 40, 41, 42, 43, 44, 45

M_SDAT	2, 17

M_SREL	4, 5, 6, 42

M_UNIT	3

M_VDAT	2, 73

Magnetic data	9

Magnetic variation	9

MAGVAR	9

Mangrove	12

Marabout	28

MARCUL	59, 66

Marine farms	66

Marine nature reserves	67

Marine services	79

Maritime areas

Various	67

Maritime jurisdiction areas	60

Marsh	20

Masked line	15, 37, 50

Mast	27, 28, 44, 45, 79

Measured distance	54

Memorial Plaque	28

Microwave Tower	28

Military practice areas	61

Minaret	28

Minefields	61, 62

MIPARE	59, 61

Mole	14

Monument	28

Mooring	50

Buoys	51

Small craft	50

Trots	51

Mooring / warping facilities	18

Mooring Mast	28

MORFAC	18, 51, 67, 71

Mosque	28

N

National territory	60

Natural sceneries	20

navigable	12, 14

Navigation Lines	77, 78, 83

Navigational marks	67

NAVLNE	52, 53, 54

O

Obelisk	28

Observation Tower	28

OBSTRN	2, 18, 44, 45, 46, 48, 51,

53, 63, 64, 66

Obstructions	44, 45, 46, 75

Offshore platforms	64

Offshore production areas	65

Offshore tanker loading systems	65

OFSPLF	64, 65, 67

Oil and gas fields	64

Oil barriers	30

OILBAR	30

OSPARE	59, 65

Overfalls	46

P

Pagoda	28

Picture files	30

PILBOP	80

Piles	18

Pilot boarding places	80

Pilot Lookout	28, 79

Pilot Office	15, 28, 79

Pilot stations	79

Ashore	79

PILPNT	18, 67, 73, 76

PIPARE	59, 64

Pipeline areas	64

Pipelines

Areas	63, 64

Disused	63

Land	63

Overhead	64

Submarine	63

PIPOHD	64

PIPSOL	19, 63

Planned construction	20, 62, 63, 64

Police Station	28

PONTON	18, 19, 33, 34, 42

Pontoons	18

Position

Quality	5

Post Office	28

Power Station	26, 28

PRCARE	54, 56, 57, 59

PRDARE	19, 26

Precautionary area	56

Production areas	26

Production information	3

PYLONS	25, 30

Pylons and bridge supports	30

Q

QTG service	77

R

Races	46

Radar	28, 78, 79, 82

Beacons	78

Conspicuous object	79

Range	78

Reference lines	78

Reflector	63

Station	78

Surveillance systems	78

transponder beacons	 See Beacons: radar transponder

Radar Dome	28, 79

Radar Tower	28

Radio

Calling-in points	79

direction-finding stations	77

Mast	28

Reporting points	79

stations	77

Tower	29

Radiobeacons

Aeronautical	77

circular	77

Directional	77

Marine and aero-marine	77

RADLNE	78

RADRFL	63, 79

RADRNG	78

RADSTA	67, 78, 79

Railway Station	23, 29

Railways	23

RAILWY	19, 23

Range systems

Relationship	53

Rapids	21

RAPIDS	21

RCRTCL	57

RCTLPT	57

RDOCAL	79

RDOSTA	67, 77

Recommended direction of traffic flow	57

Recommended routes	57

Recommended tracks and routes	52

RECTRC	52, 53

Refinery	26

Regulated areas	59

Relationship	52, 54, 68, 83

Anchorage	51

Lighthouses	67

RESARE	50, 51, 58, 59, 62, 65, 66, 67

Rescue stations	67, 80

Restricted areas	59, 66

RETRFL	67, 72

Retro-reflectors	72

RIVBNK	21, 40

River bank	14, 21

Rivers	12, 21, 40

RIVERS	21, 37, 40, 82

Roads and tracks	24

ROADWY	24

Rocks

Do not cover	43

May cover	43

Rocks and reefs	43

Routeing measures	54

Routes

Deep water	56

Recommended	57

Routing measures	54

Two-way	58

RSCSTA	80, 81

RTPBCN	67, 78

Ruin	29

RUNWAY	25

S

Safety zones	65

Saltpans	22

Sand waves	41, 48

SBDARE	48

SCAMIN	6

Sea areas	49

SEAARE	17, 20, 21, 22, 23, 40, 49, 82

Seabed

Description of bottom	48

Nature	48

Special bottom types	48

Seaplane landing areas	25, 66

Seasonal Objects	3

Sector Lights	73

Settlement	26

Shoreline Construction	35

Sign	31

Signal stations	79

SILTNK	26, 27, 28, 29

SISTAT	67, 81

SISTAW	67, 81

slag heap	26

SLCONS	14, 15, 17, 19, 20, 21, 22, 23,

24, 40, 43, 67

SLOGRD	20, 21, 23

SLOTOP	21, 23, 24

Small craft facilities	16

SMCFAC	16

SNDWAV	41, 48

SOUNDG	2, 32, 41, 45

Soundings	32, 45

Accuracy	5

Drying	33

Enclosed by danger line	46

Existence doubtful	32

Geometry	33

out of position	33

Quality	5

Technique	5

Speed limits	50

Spire	29

SPLARE	25, 59, 66

Spoil ground	62

Spoil grounds	65

Spot Heights	2, 20

SPRING	49

Springs in seabed	49

SQUARE	26

Stockpile	26

STSLNE	61

Submarine

Cable Areas	63

Cables	See Submarine Cables

Pipeline	63

Transit lanes	62

SUBTLN	59, 62

Survey reliability	5

Swept areas	41

Swinging circle	51

SWPARE	41

T

T_HMON	10, 11

T_NHMN	11

T_TIMS	10, 11

Tank	27

Farm	26

Tanker loading buoys	65

Television Mast	29

Television Tower	29

Territorial sea	60, 61

TESARE	59, 61

Text	7

Tides

Data	10

Harmonic	10, 11

Non-harmonic	11, 12

Stream (flood/ebb)	11

Stream data	11

Stream panels	12

Stream time series	11

Time series data	10, 11

Tideway	49

TIDEWY	49

Timber yards	19, 26

Time varying objects	9

Timeball Tower	29

TOPMAR	67, 72

Topmarks	72

Topography	12

Tower	27, 28, 29, 70, 78, 79, 80

Towers

Church	27

Clock	27

Control	25, 27, 82

Cooling	27

Lookout	28

Microwave	28

Observation	28

Radar	28

Radio	29

Television	29

Timeball	29

Water	29

Town	26

Traffic Separation Scheme	54, 55, 57

Boundaries	55

Crossings	55

Lanes	54

Roundabouts	55

Separation line	55

Separation zone	55

Systems	57

Traffic signal station	See

Traffic zone

Inshore	56

Transit Lines	See Lines - Transit

Transit Shed	29

TS_FEB	11

TS_PAD	12

TS_PNH	12

TS_PRH	11, 12

TS_TIS	11, 12

TSELNE	54, 55, 57

TSEZNE	54, 55, 57

TSSBND	54, 55, 57

TSSCRS	54, 55, 56, 57, 59

TSSLPT	54, 55, 56, 57, 59

TSSRON	54, 55, 56, 57, 59

TUNNEL	23

Tunnels	23

Two-way routes	58

TWRTPT	58

U

Under construction	19, 62, 63, 64

Undersea features	49

Units	3

UNSARE	22, 33, 34, 35, 37, 40, 41,

42, 45, 46

Unsurveyed areas	41

UWTROC	2, 33, 43, 44, 48, 53

V

VEGATN	22

Vegetation	22

Vertical measurements	13

Views and sketches	30

W

Walls	See Fences and walls

Warehouse	29

Warning signal station	81

Water Tower	29

Waterfalls	21, 22

WATFAL	22

WATTUR	46

WEDKLP	49

Weed - kelp	49

Wellheads	45, 64

Wharf	15

Wind farm	26

Windmill	29

Wrecks	44, 46

Foul areas and obstructions	44

Historic	66

WRECKS	2, 44, 45, 46, 53

Z

Zones

Contiguous	61

Exclusive economic	61

Fishery	61

Safety	65

__________

Addendum to S-57 Edition 3.1 Appendix B.1 Annex A

Use of the Object Catalogue Edition 2.0

This addendum has been produced to inform users of changes agreed at the 5th TSMAD WG meeting held in April 2000, during the S-57 Ed 3.1 familiarisation period. They have not been included in Edition 2.0 of the Use of the Object Catalogue (UoOC) in order to maintain consistency between the familiarisation and the official copies of Ed 2.0 adopted in November 2000. These changes will be included in the text of the next edition of the UoOC.

#### 2.2.3.1	Quality of bathymetric data

Change the first bullet from:

- The attributes POSACC, SOUACC and TECSOU may be used to upgrade the components that are of higher accuracy.

to

- The attributes POSACC, SOUACC and TECSOU may be used to upgrade the components that are of higher accuracy or different technique

Add the following bullet:

- M_QUAL may be encoded exclusively over wet areas or a single polygon may be created for the whole cell.

2.2.3.3  		Quality of sounding

Change the first sentence from:

- The quality of sounding is encoded with QUASOU, when different to the meta object M_SREL.

- to

- The quality of sounding is encoded with QUASOU, when different to the meta object M_SREL, see tables 5.1 and 6.4.

#### 2.2.3.4	 Sounding accuracy

Change the first sentence from:

- The quality of sounding is encoded with QUASOU, when different to the meta object M_SREL.

## to

- The accuracy of sounding is encoded with SOUACC, when different to the meta object M_QUAL. If SOUACC is encoded for M_QUAL, refer to the remarks in clause 2.2.3.1 and 5.6.

### 2.2.7		Use of the attribute SCAMIN

Change the fourth bullet of the “Remarks” from:

- Group1 objects must always be displayed. Therefore, SCAMIN must not be used with Group1 objects.

to

- Group1 and META objects must always be displayed. Therefore, SCAMIN must not be used with Group1 and META objects.

### 2.6.1		Issuing updates in advance

Change paragraphs a), b), c) and d) to read as follows:

a) 	If the future update information contained in the update message involves the addition of objects to the existing data (e.g. a new lighthouse), the attribute DATSTA on any new object  must contain the date at which the update information becomes active.

b) If the future update information contained in the update message involves the modification of existing objects (e.g. a change in a traffic routing system), this must be treated as a deletion of the existing objects and replacement with new objects. See a) above and c) below.

c) If the future update information contained in the update message involves the deletion of existing objects (e.g. the removal of a buoy), the update message should set the DATEND attribute of all objects to be deleted to the date at which the update information  becomes active.

d) To highlight to the mariner that the update information contained in an update message will take place in the future, it is recommended that a caution area (CTNARE) is created covering the location at which  the future update will take place. A warning note specifying, in plain language, the nature of the future change should be encoded either in INFORM or in a file referenced by the TXTDSC attribute of the CTNARE. The DATEND attribute of the CTNARE must be set to the date at which the update information becomes active.

## 4.1				Land area

### Change the second sentence from:

Rivers, canals, lakes, basins and docks which are not navigable at compilation scale must be encoded on top of a LNDARE.

To:

Rivers, canals, lakes, basins and docks which are not navigable at compilation scale must be encoded on top of a LNDARE or UNSARE.

#### 4.6.7.1	Mooring / warping facilities (see M4 - §327)

Change the third bullet of the “Remarks” from:

- Area MORFAC must be covered by an area LNDARE.

to

- Area MORFAC with the attribute value WATLEV different from 3, 4 or 5 must be covered by an area LNDARE.

### 4.6.8		Hulks

Delete the last bullet of the “Remarks” section which is as follows:

- Floating breakwaters are encoded as HULKES with CATHLK = 5.

#### 4.7.7.1	 Rapids

Change the third line from:

The area covered by rapids is also covered by an object RIVERS and an object LNDARE.

to

The area covered by rapids is also covered by an object RIVERS and an object LNDARE, or UNSARE.

### 4.8.5		Dams (see M4 - §364.2)

Change the first bullet of the “Remarks” paragraph from:

- Dams are encoded with a LNDARE underneath.

to

- When a DAMCON is of type area, it must have a LNDARE underneath.

### 4.8.15		Buildings, landmarks, tanks, silos (see M4 - §370 to §378)

Change the existing sentence prior to table 4.1 from:

In the following table the symbol ‘/’ means that this attribute is not relevant. A blank means that the encoder can chose a value.

to

In the following table the symbol ‘/’ means that this attribute is not relevant. A blank means that the encoder can chose a value. The table contains the most common examples of coding; other coding combinations are possible.

### 4.8.18		Pylons and bridge supports (see M4 - §381 and §382)

Change the first bullet of the “Remarks” section from:

- Area PYLONS must be covered by an area LNDARE. See also clause 4.8.10.

to

- Area PYLONS with the attribute value WATLEV different from 3, 4 or 5, must be covered by an area LNDARE. See also clause 4.8.10.

5.4.3		Use of DRVAL1 and DRVAL2 for depth areas in general.

Change paragraph number 9. From

- 9.	If the area is limited by an incomplete depth contour an one side (such as in incompletely surveyed area), and a standard depth contour on the other:

DRVAL1 takes the value of the shallowest depth contour.

DRVAL2 takes the value of the deepest depth contour.

Note: The encoding of this DEPARE as an area is optional - see figure 5.

to

- 9.	If the area is limited by an incomplete depth contour on one side (such as in an incompletely surveyed area), and a complete depth contour on the other:

DRVAL1 takes the value of the shallowest depth contour.

DRVAL2 takes the value of the deepest depth contour.

Note: The encoding of this DEPARE as an area is optional - see figure  6.

6.1.2		Rocks which may cover

Change NATSUR from:

NATSUR -	9 - rock

14 – coral

to

NATSUR -	9 - rock

14 - coral

18 – boulder

### 6.2.2		Obstructions and foul areas

Change the Remarks section from:

Remarks:

- If the nature of a dangerous underwater object, dangerous underwater area, or floating object is not explicitly known, it must be encoded as an OBSTRN.

- Area OBSTRN must be covered by an area object from Group 1 (DEPARE or UNSARE as appropriate).

to

Remarks:

- If the nature of a dangerous underwater object, dangerous underwater area, or floating object is not explicitly known, it must be encoded as an OBSTRN.

- Area OBSTRN must be covered by an area object from Group 1 (LNDARE, DEPARE or UNSARE as appropriate).

- In certain circumstances where an obstruction is always dry (e.g. cribs), it may be covered by a LNDARE.

### 9.1.2		Speed limits (see M4 - §430.2)

Change the first sentence from:

Speed is often limited inside harbours to prevent wakes. This is encoded as RESARE with CATREA = 24 (no wake area) and RESTRN = 13 (no wake).

to

Speed is often limited inside harbours to prevent wakes. This is encoded as RESARE with CATREA = 24 (no wake area) or RESTRN = 13 (no wake).

### 9.2.3		Anchoring restricted (see M4 - §431.4)

Change the first paragraph from:

Areas in which anchoring is prohibited or restricted are encoded as RESARE (see clause 11.1) with RESTRN = 1 (anchoring prohibited) or 2 (anchoring restricted). Additional information about the restriction is encoded in INFORM (or TXTDSC).

to

Areas in which anchoring is prohibited or restricted are encoded by RESARE or by other objects with the attribute RESTRN (see clause 11.1)  where RESTRN = 1 (anchoring prohibited) or 2 (anchoring restricted). Additional information about the restriction is encoded in INFORM (or TXTDSC).

## 12.9		Radio stations (see M4 - §480 to §485)

## Change the last bullet of the “Remarks” section from:

- Further information e.g. frequencies, transmission characteristic may be encoded using the attribute INFORM (or TXTDSC).

## to

- Further information e.g. transmission characteristic may be encoded using the attribute INFORM (or TXTDSC).

# 14.			Geographic names

## Add the following paragraphs after the existing paragraphs:

Geographical names will be coded in the OBJNAM attribute based on the following criteria and at the HO’s discretion:

- Point names (area or point) that do not show navigational aids may be coded with a LNDRGN object and the geographic name coded in the OBJNAM attribute.

- Point names that show one navigational aid may be coded in the OBJNAM attribute of the master object associated with the navigational aid; if more than one navigational resides on the point, create a LNDRGN area object and encode the geographic name in the OBJNAM attribute of the LNDRGN object.

- Point SBDARE objects associated with a geographic name will be used to create a SEAARE point object and the geographic name entered in the OBJNAM attribute of the SEAARE point object.

- Major islands and islands close to the primary shipping corridors may be coded with the LNDARE object delimiting the island(s) and the OBJNAM attribute.

- A selection of points, bays and shoals listed in HO’s Sailing Directions that may assist in navigating may be coded with one of LNDRGN, UWTROC, SBDARE, SEAARE or OBSTRN object and the OBJNAM attribute.

- Administrative areas of international, national, provincial or municipal jurisdiction to the extent that these may have legal inference may be coded with the ADMARE object and the OBJNAM attribute.

- Major cities along the coast may be coded with a BUAARE object and the OBJNAM attribute.

- Names of navigable rivers, lakes and canals may be coded with a SEAARE object and the OBJNAM attribute.

In all instances, if the exact extent of the feature to be named is known, an area object must created. In other cases, an existing point feature will be used or created to enter the geographical name associated with an appropriate object.

# 15.			Collection objects

- Add the following paragraph to the end of this section:

If a collection object extends beyond a cell boundary (i.e. the objects that make up the collection are spread over multiple cells), the collection object should be repeated in each cell that contains one or more component objects. However, only the objects that exist in the cell that contains the instance of the collection object can be referenced by that collection object. If this technique is used, each instance of the original collection object must have the same feature object identifier (LNAM). It is up to the application (e.g. the ECDIS) that uses the cells to rebuild the complete collection object based on the unique feature object identifier.

It is highly recommended that no use be made of pointers that reference objects outside the cell in which the pointer is encoded.  Use of such pointers can not be prohibited as no such rule exists in the ENC Product Specification.

__________